Structure Padding &

#pragma pack

A deep-dive reference for embedded & systems engineers

Covers: alignment rules · padding bytes · sizeof · cache effects · pragma pack · __attribute__((packed))

# 1. Why Memory Alignment Exists

Memory alignment is the constraint that a variable of type T must reside at a memory address that is a multiple of T's alignment requirement. This requirement comes from the CPU's bus architecture: most processors can fetch data from memory in one bus transaction only if the data is naturally aligned. An unaligned access either triggers a hardware exception or requires the CPU to issue two separate memory transactions, halving throughput.

The C/C++ compiler inserts silent padding bytes between structure members to satisfy these requirements automatically. Understanding padding is critical for:

- Minimising struct size in memory-constrained embedded systems

- Matching hardware register maps or network/protocol headers exactly

- Maximising cache performance by fitting more objects in a cache line

- Correctly using memcpy / DMA for binary serialisation

- Avoiding undefined behaviour caused by unaligned pointer dereferences

# 2. Fundamental Alignment Rules

★ KEY RULE On most 32-bit and 64-bit architectures: char aligns to 1, short to 2, int/float to 4, double/pointer to 8. The compiler pads members so each starts at its alignment boundary, then pads the struct tail so its total size is a multiple of the largest member's alignment.

| Type | Size (bytes) | Alignment (bytes) | Architecture |
| --- | --- | --- | --- |
| char / uint8_t | 1 | 1 | All |
| short / uint16_t | 2 | 2 | All |
| int / uint32_t | 4 | 4 | 32-bit & 64-bit |
| float | 4 | 4 | All |
| long (32-bit) | 4 | 4 | 32-bit Linux/Windows |
| long (64-bit) | 8 | 8 | 64-bit Linux |
| double | 8 | 8 | All |
| pointer (32-bit) | 4 | 4 | 32-bit |
| pointer (64-bit) | 8 | 8 | 64-bit |

Table 1 – Common type sizes and alignment requirements (x86/ARM)

# 3. Example 1 — Basic Structure Padding

## 3.1  The struct definition

basic_struct.c

struct Basic {

char   a;    /* 1 byte  – offset 0 */

int    b;    /* 4 bytes – needs 4-byte alignment */

char   c;    /* 1 byte  – offset 8 */

};

printf("%zu\n", sizeof(struct Basic));  /* prints 12, not 6 */

## 3.2  Memory layout (12 bytes)

| Byte offset | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | a | PAD | PAD | PAD | b0 | b1 | b2 | b3 | c | PAD | PAD | PAD |

Red cells = compiler-inserted padding bytes

ℹ NOTE After 'a' (offset 0), 'b' needs to start at a multiple of 4. That forces 3 pad bytes (offsets 1–3). After 'c' (offset 8), the struct total must be a multiple of 4 (the largest member alignment), adding 3 more tail pad bytes.

# 4. Example 2 — Eliminating Padding by Reordering

A simple rule: order members largest-first. This completely eliminates all padding in Example 1:

optimised_struct.c

struct Optimised {

int    b;    /* 4 bytes – offset 0 */

char   a;    /* 1 byte  – offset 4 */

char   c;    /* 1 byte  – offset 5 */

/* tail pad: 2 bytes to reach multiple of 4 */

};

printf("%zu\n", sizeof(struct Optimised));  /* prints 8 */

| Byte offset | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | b0 | b1 | b2 | b3 | a | c | PAD | PAD |

2 bytes of unavoidable tail pad (struct must be size % 4 == 0) vs 6 bytes in the naive version

# 5. Example 3 — Nested Structs

Nested structs follow the same rules. The inner struct's alignment bubbles up and determines the outer struct's padding requirements.

nested_struct.c

struct Inner {

char  x;   /* 1 byte  – offset 0 */

int   y;   /* 4 bytes – offset 4 (3 pad bytes inserted) */

};             /* sizeof == 8, alignment == 4 */

struct Outer {

char        flag;    /* offset 0 */

struct Inner inner;  /* needs 4-byte align → offset 4 */

short       val;     /* offset 12 */

};             /* sizeof == 16 */

| Byte offset | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | flag | PAD | PAD | PAD | x | PAD | PAD | PAD | y0 | y1 | y2 | y3 | val0 | val1 | PAD | PAD |

Outer struct: 16 bytes. Without nesting overhead, members sum to 8 bytes.

# 6. Example 4 — Double and Pointer Alignment (64-bit)

mixed_types.c

struct MixedTypes {

char    a;      /* 1 byte  – offset 0  */

double  d;      /* 8 bytes – offset 8 (7 pad bytes) */

int     i;      /* 4 bytes – offset 16 */

char    b;      /* 1 byte  – offset 20 */

};                  /* sizeof == 24 */

/* Offsets confirmed with: */

printf("offsetof a=%zu d=%zu i=%zu b=%zu\n",

offsetof(struct MixedTypes, a),

offsetof(struct MixedTypes, d),

offsetof(struct MixedTypes, i),

offsetof(struct MixedTypes, b));

| Offset | 0 | 1-7 | 8-15 | 16-19 | 20 | 21-23 |
| --- | --- | --- | --- | --- | --- | --- |
| Member | a | PAD (7) | d (8 bytes) | i (4 bytes) | b | PAD (3) |

7 padding bytes between 'a' and 'd' — the most expensive padding possible

# 7. Bitfields and Padding

Bitfields allow packing multiple flags into a single integer, but they still carry alignment overhead at the container level.

bitfield_struct.c

struct StatusReg {

uint32_t error   : 1;   /* bit 0 */

uint32_t busy    : 1;   /* bit 1 */

uint32_t ready   : 1;   /* bit 2 */

uint32_t         : 5;   /* bits 3–7: unnamed pad bits */

uint32_t code    : 8;   /* bits 8–15 */

uint32_t         : 16;  /* bits 16–31: reserved */

};                          /* sizeof == 4 */

/* WARNING: bitfield layout is IMPLEMENTATION-DEFINED in C.  */

/* For hardware registers, always use explicit shift/mask.    */

⚠ WARNING Bitfield ordering (LSB-first vs MSB-first) is implementation-defined. Two compilers or two architectures may lay out the same bitfield struct differently. For hardware register maps, prefer explicit bitmasks and shifts rather than bitfields.

PART 2 — #pragma pack

# 8. What is #pragma pack?

#pragma pack is a compiler directive that overrides the default alignment rules and forces members to be packed at specified byte boundaries. Its primary use cases are:

- Network / protocol headers – fields must map exactly to wire bytes

- Hardware register maps – MMIO structs must match silicon layout

- Binary file formats – reading/writing structs directly to disk

- Shared memory IPC – struct layout must be identical across processes/compilers

- Memory reduction – squeeze structs in constrained embedded RAM

## 8.1  Syntax variants

pragma_syntax.c

#pragma pack(n)          /* set alignment to n bytes (1,2,4,8…) */

#pragma pack()           /* restore default alignment            */

#pragma pack(push, n)    /* push current alignment, set n        */

#pragma pack(pop)        /* restore previously pushed alignment  */

/* GCC / Clang attribute equivalent (single struct): */

struct Foo { int x; char y; } __attribute__((packed));

/* MSVC __declspec alternative: */

/* Not standard; prefer #pragma pack(push/pop)      */

⚠ WARNING Always use push/pop pairs instead of bare #pragma pack(n)/#pragma pack(). A bare pack without push/pop silently changes alignment for ALL subsequent structs in the translation unit, breaking third-party headers included after yours.

# 9. Example 5 — #pragma pack(1): Fully Packed

network_header.c

#pragma pack(push, 1)         /* no padding whatsoever */

struct NetworkHeader {

uint8_t  version;         /* offset 0, size 1 */

uint16_t length;          /* offset 1, size 2 */

uint32_t src_ip;          /* offset 3, size 4 */

uint32_t dst_ip;          /* offset 7, size 4 */

uint8_t  flags;           /* offset 11, size 1 */

};                            /* sizeof == 12 */

#pragma pack(pop)

| Offset | 0 | 1-2 | 3-6 | 7-10 | 11 |
| --- | --- | --- | --- | --- | --- |
| Member | version | length | src_ip | dst_ip | flags |

With pack(1): sizeof == 12. Without it: sizeof would be 16 (4 bytes wasted on padding before src_ip and flags)

⚠ WARNING Accessing 'src_ip' at offset 3 (non-4-byte-aligned) on ARM Cortex-M0/M0+ will cause a HardFault. On x86 it works but costs extra clock cycles. Always use memcpy or byte-by-byte accessors for packed structs: uint32_t val; memcpy(&val, &hdr.src_ip, 4);

# 10. Example 6 — #pragma pack(2): Half-Aligned

pack2_struct.c

#pragma pack(push, 2)

struct SensorData {

char    id;       /* offset 0, size 1 */

int     value;    /* offset 2 (pad 1), size 4 */

double  reading;  /* offset 6 (no extra pad), size 8 */

char    unit;     /* offset 14, size 1 */

};                    /* sizeof == 16 */

#pragma pack(pop)

/* Compare with default (no pragma): sizeof == 24 */

| Member | Default offset | pack(2) offset | Savings |
| --- | --- | --- | --- |
| id (char) | 0 | 0 | — |
| [pad] | 1–3 (3 bytes) | 1 (1 byte) | 2 bytes |
| value (int) | 4 | 2 | — |
| reading (double) | 8 | 6 | — |
| unit (char) | 16 | 14 | — |
| [tail pad] | 17–23 (7 bytes) | 15 (1 byte) | 6 bytes |
| Total sizeof | 24 | 16 | 8 bytes saved |

Comparison: default alignment vs pack(2)

# 11. Example 7 — Nested Structs with Selective Packing

A common pattern in driver code: pack only the hardware-facing struct, leave the software-only struct at natural alignment.

usb_driver_struct.c

/* Hardware register block – must match silicon exactly */

#pragma pack(push, 1)

struct HW_UsbRegs {

uint8_t  ctrl;      /* offset 0 */

uint16_t status;    /* offset 1 */

uint32_t addr;      /* offset 3 */

uint8_t  ep_cfg[4]; /* offset 7 */

};                      /* sizeof == 11 */

#pragma pack(pop)

/* Software context – natural alignment for performance */

struct UsbDriver {

struct HW_UsbRegs __iomem *regs; /* pointer: 8 bytes */

uint32_t                   irq;  /* 4 bytes           */

uint32_t                   flags;/* 4 bytes           */

uint8_t                    state;/* 1 byte            */

};  /* sizeof == 24 (natural alignment, no pack) */

✔ TIP The UsbDriver struct uses a pointer to HW_UsbRegs, not an embedded copy. This is the canonical Linux driver pattern: keep MMIO structs packed, keep driver state structs naturally aligned.

# 12. Example 8 — Binary File Format (BMP Header)

bmp_header.c

#pragma pack(push, 1)

struct BmpFileHeader {

uint16_t signature;   /* 'BM' = 0x424D   offset 0  */

uint32_t file_size;   /* total file size  offset 2  */

uint16_t reserved1;   /* must be 0        offset 6  */

uint16_t reserved2;   /* must be 0        offset 8  */

uint32_t data_offset; /* pixel data start offset 10 */

};                        /* sizeof == 14 */

struct BmpInfoHeader {

uint32_t header_size; /* offset 0  */

int32_t  width;       /* offset 4  */

int32_t  height;      /* offset 8  */

uint16_t planes;      /* offset 12 */

uint16_t bpp;         /* offset 14 */

uint32_t compression; /* offset 16 */

uint32_t image_size;  /* offset 20 */

int32_t  x_ppm;       /* offset 24 */

int32_t  y_ppm;       /* offset 28 */

uint32_t colors_used; /* offset 32 */

uint32_t colors_imp;  /* offset 36 */

};                        /* sizeof == 40 */

#pragma pack(pop)

/* Read header directly from file: */

struct BmpFileHeader hdr;

fread(&hdr, sizeof(hdr), 1, fp);

# 13. GCC/Clang: __attribute__((packed))

__attribute__((packed)) is the GCC/Clang equivalent of #pragma pack(1) applied to a single struct. It is more portable for Linux kernel development since it avoids the push/pop boilerplate.

ethernet_packed.c

struct __attribute__((packed)) EthernetFrame {

uint8_t  dst_mac[6];    /* offset 0,  size 6 */

uint8_t  src_mac[6];    /* offset 6,  size 6 */

uint16_t ether_type;    /* offset 12, size 2 */

uint8_t  payload[1500]; /* offset 14         */

};  /* sizeof == 1514 — exactly the 802.3 frame wire size */

/* Linux kernel equivalent using kernel macro: */

struct EthernetFrame {

uint8_t  dst_mac[6];

uint8_t  src_mac[6];

__be16   ether_type;

uint8_t  payload[1500];

} __packed;

# 14. Increasing Alignment: alignas / __attribute__((aligned))

Alignment can also be increased beyond the natural minimum — useful for SIMD (SSE/AVX/NEON) data, cache-line alignment, and DMA buffers that require page alignment.

aligned_examples.c

#include <stdalign.h>   /* C11 */

/* Force struct to start on a 64-byte cache line boundary */

struct alignas(64) CacheAligned {

uint64_t counter;   /* frequently read — no false sharing */

uint8_t  pad[56];   /* manual padding to fill cache line */

};

/* AVX-512 SIMD buffer – must be 64-byte aligned */

float __attribute__((aligned(64))) avx_buffer[16];

/* DMA-coherent buffer – Linux kernel */

dma_addr_t dma_handle;

void *cpu_addr = dma_alloc_coherent(dev, PAGE_SIZE,

&dma_handle, GFP_KERNEL);

✔ TIP Cache-line alignment is critical in multi-core systems to prevent 'false sharing' — where two CPU cores repeatedly invalidate each other's cache lines because two independent variables happen to share the same 64-byte cache line.

# 15. Common Mistakes and Anti-patterns

| Mistake | Consequence | Fix |
| --- | --- | --- |
| Assuming sizeof(struct) == sum of member sizes | Wrong buffer allocation, protocol breakage | Always use sizeof(struct) |
| Using #pragma pack without push/pop | Corrupts alignment of all subsequent structs | Always push/pop around packed region |
| Taking pointer to packed member | Unaligned pointer → undefined behaviour / crash on ARM | Use memcpy to extract the value |
| Comparing packed structs with memcmp | Pad bytes contain garbage — false inequality | Zero-init with memset or compare field-by-field |
| Ignoring endianness with packed network structs | Wrong byte order on little-endian hosts | Use htonl/ntohl or __be32 kernel types |
| Bitfields for hardware registers | Layout is implementation-defined | Use volatile uint32_t + explicit masks |

Table 2 – Common structure padding mistakes

# 16. Inspecting Layout: sizeof and offsetof

offsetof_demo.c

#include <stddef.h>

struct Example {

char   a;

int    b;

double c;

char   d;

};

int main(void) {

printf("sizeof(Example) = %zu\n", sizeof(struct Example));

printf("offsetof(a) = %zu\n",    offsetof(struct Example, a));

printf("offsetof(b) = %zu\n",    offsetof(struct Example, b));

printf("offsetof(c) = %zu\n",    offsetof(struct Example, c));

printf("offsetof(d) = %zu\n",    offsetof(struct Example, d));

return 0;

}

/* Output (64-bit Linux):              */

/* sizeof(Example) = 24               */

/* offsetof(a) = 0                    */

/* offsetof(b) = 4                    */

/* offsetof(c) = 8                    */

/* offsetof(d) = 16                   */

Tip: The Linux kernel's container_of() macro relies on offsetof() to recover a pointer to the outer struct from a pointer to a nested member — a pattern used heavily in linked-list-based driver code.

# 17. Quick Reference Summary

| Scenario | Directive / Attribute | sizeof effect | Risk |
| --- | --- | --- | --- |
| Default (no pragma) | — (compiler default) | Largest | None — safest |
| Fully packed | #pragma pack(1) or __attribute__((packed)) | Smallest | Unaligned access faults on strict-align CPUs |
| Half-aligned | #pragma pack(2) | Moderate | Partial — 2-byte accesses OK, 4-byte may misalign |
| Increase alignment | alignas(N) or __attribute__((aligned(N))) | Same or larger | None — adds padding, never removes it |
| Single member override | _Alignas(N) on member | Struct may grow | None |
| Cache-line padding | alignas(64) + manual pad array | 64 bytes min | Wastes memory if many objects |

Table 3 – Alignment directive quick reference

# 18. Final Notes for Embedded / Driver Engineers

- USB/HDMI descriptors and DDR SPD data use tightly packed binary layouts. Always use #pragma pack(1) or __packed when mapping C structs to these.

- Ethernet MAC / IP headers accessed in interrupt context should use __packed + accessor functions to avoid misaligned pointer warnings in sparse/cppcheck.

- Avoid taking the address of a packed member — the resulting pointer has reduced alignment and gcc -Waddress-of-packed-member warns about this.

- GCC -Wpadded flag reports every padding byte added to a struct — run this during review to catch unexpected layout bloat.

- pahole tool (part of dwarves) analyses kernel DWARF debug info and shows exact struct layouts, padding holes, and cache line utilisation.

- C11 static_assert lets you embed compile-time checks: static_assert(sizeof(struct Hdr)==14, "BMP header size mismatch");

★ KEY RULE Rule of thumb for embedded structs: (1) order members largest-to-smallest to minimise natural padding. (2) Use #pragma pack only when the layout must match external binary data. (3) Always document why a struct is packed, and add a static_assert on its size.