2_STM32F401x_Ref_Manual (1).pdf

PDF

CortexM4_STM32_Programming_Manual.pdf

PDF

1_STM32F401x_Datasheet.pdf

PDF

Raayanmini_v4.0_4.1_Schematic.pdf

PDF

based on these explain the chapter Create concise professional notes for: just read this pdf and i dont know about anything aboutCreate concise professional notes for: "STM32F401 SPI Protocol" Cover only: - SPI basics - Master/slave - MOSI/MISO/SCK/NSS - CPOL/CPHA - SPI registers - Full duplex communication - SPI modes Include: - SPI timing diagrams - SPI communication diagrams - register explanations - bare-metal SPI code - SPI sensor example explain this things

Perfect. The SPI_DR (SPI Data Register) is the heart of SPI communication.

Think of it as a mailbox between your program and the SPI hardware.

Your Code
    |
    v
 SPI_DR
    |
    v
SPI Hardware
    |
    v
MOSI/MISO Pins

# What is SPI_DR?

SPI_DR = SPI Data Register

Used for:

- Writing data to transmit

- Reading received data

SPI1->DR

# Internal View

Inside SPI peripheral:

SPI Peripheral

           +------------+
           |  SPI_DR    |
           +------------+
                  |
                  v
          +---------------+
          | Shift Register|
          +---------------+
                  |
            MOSI / MISO

The CPU talks to SPI_DR.

The SPI hardware talks to the shift register.

# Transmission Example

Suppose:

SPI1->DR = 0xA5;

What happens?

## Step 1

CPU writes:

SPI_DR = 0xA5

SPI_DR

10100101

## Step 2

SPI hardware copies data from DR to Shift Register.

SPI_DR
   |
   v
Shift Register

## Step 3

Clock starts.

SCK

_|-|_|-|_|-|_|-|_|-|_|-|_|-|_

## Step 4

Bits leave one by one.

MOSI

1 0 1 0 0 1 0 1

After 8 clocks:

0xA5 transmitted

# Receive Example

Slave sends:

0x3C

00111100

## Step 1

Bits arrive on MISO.

MISO

0 0 1 1 1 1 0 0

## Step 2

Shift Register collects them.

Shift Register

00111100

## Step 3

After 8 bits received:

Shift Register
      |
      v
SPI_DR

Now:

SPI_DR = 0x3C

## Step 4

CPU reads:

data = SPI1->DR;

Now:

data = 0x3C;

# Why Wait For TXE?

Before writing DR:

while(!(SPI1->SR & (1<<1)));

This checks:

TXE = Transmit Buffer Empty

Meaning:

SPI_DR is empty

and ready for new data.

## Example

Wrong:

SPI1->DR = 0x11;
SPI1->DR = 0x22;

The second write may overwrite data before transmission completes.

Correct:

while(!(SPI1->SR & (1<<1)));

SPI1->DR = 0x11;

# Why Wait For RXNE?

After receiving:

while(!(SPI1->SR & (1<<0)));

Checks:

RXNE = Receive Buffer Not Empty

Meaning:

SPI_DR contains received data

Then:

data = SPI1->DR;

# Full Duplex Example

Suppose:

Master sends:

0xA5

Slave sends:

0x3C

at the same time.

## Before Clock Starts

Master:

SPI_DR = 0xA5

Slave:

SPI_DR = 0x3C

## During Clocks

Master MOSI ---> 10100101

Master MISO <--- 00111100

## After 8 Clocks

Master DR contains:

0x3C

Slave DR contains:

0xA5

Both exchanged data simultaneously.

# Important Concept

When you write:

SPI1->DR = 0x55;

you are not directly writing to MOSI pin.

Instead:

CPU
 |
 v
SPI_DR
 |
 v
Shift Register
 |
 v
MOSI Pin

# Interview Question

### What is SPI_DR?

Answer:

SPI_DR (SPI Data Register) is the register used to transmit and receive SPI data.

- Writing to SPI_DR loads data into the SPI transmitter.

- Reading from SPI_DR retrieves received data.

- The SPI hardware automatically moves data between SPI_DR and the internal shift register during SPI transfers.

### Typical SPI Send

while(!(SPI1->SR & (1<<1)));  // TXE

SPI1->DR = 0x55;

### Typical SPI Receive

while(!(SPI1->SR & (1<<0)));  // RXNE

data = SPI1->DR;

A useful mental picture is:

SPI_DR = Mailbox
Shift Register = Delivery Truck
MOSI/MISO = Road

Your program puts data into the mailbox (SPI_DR), and the SPI hardware delivers it bit-by-bit on the SPI bus.

what is the speed of the spi normal and what speed in this we can use

For STM32F401, the datasheet specifies:

Up to 4 SPI interfaces with speeds up to 42 Mbit/s when the CPU runs at 84 MHz.

# How SPI Speed is Generated

SPI speed comes from the peripheral clock divided by a prescaler.

SPI Clock = Peripheral Clock / Prescaler

For STM32F401:

APB2 Clock (SPI1) = 84 MHz
APB1 Clock (SPI2/SPI3) = 42 MHz

# SPI Baud Rate Prescaler

SPI_CR1 contains BR[2:0] bits.

| BR Bits | Division |
| --- | --- |
| 000 | ÷2 |
| 001 | ÷4 |
| 010 | ÷8 |
| 011 | ÷16 |
| 100 | ÷32 |
| 101 | ÷64 |
| 110 | ÷128 |
| 111 | ÷256 |

# Example: SPI1

SPI1 clock source:

84 MHz

If:

BR = 000

then:

84 MHz / 2 = 42 MHz

SPI speed:

42 Mbit/s

This is the maximum supported SPI speed.

# Example Speeds

For SPI1 (84 MHz source):

| Prescaler | SPI Speed |
| --- | --- |
| ÷2 | 42 MHz |
| ÷4 | 21 MHz |
| ÷8 | 10.5 MHz |
| ÷16 | 5.25 MHz |
| ÷32 | 2.625 MHz |
| ÷64 | 1.312 MHz |
| ÷128 | 656 kHz |
| ÷256 | 328 kHz |

# Does Every Device Support 42 MHz?

No.

The SPI peripheral may support 42 MHz, but the slave device often cannot.

Examples:

| Device | Typical SPI Speed |
| --- | --- |
| MPU6050 | ~1 MHz |
| W25Q Flash | 50–80 MHz |
| TFT LCD | 10–40 MHz |
| ADCs | Depends on model |

You must always check the slave device datasheet.

# For Learning and Debugging

Start with:

1 MHz

or

2 MHz

because:

- Easier to debug on logic analyzer

- Reliable wiring

- Less signal integrity issues

After communication works, increase speed.

# Example Configuration

Suppose SPI1 clock is 84 MHz and you want ~5 MHz SPI.

84 MHz / 16 = 5.25 MHz

Set:

SPI1->CR1 |= (3 << 3);   // BR = 011 (divide by 16)

# Interview Question

Q: What is the maximum SPI speed of STM32F401?

A: The STM32F401 SPI peripheral can operate up to 42 Mbit/s when the MCU runs at 84 MHz and the SPI prescaler is set to divide-by-2.

For your STM32F401 learning projects, I recommend starting at 1–5 MHz, then later trying 10 MHz, 21 MHz, and finally 42 MHz to see the effect on timing and waveforms.

i have doubt to use spi or i2c can we use any gpio pin or for any specific pins are there

No, you cannot use any GPIO pin for SPI or I2C on STM32F401 (at least not when using the hardware SPI/I2C peripheral).

STM32 peripherals are connected internally only to specific pins through the Alternate Function (AF) system.

# Example: SPI1

Common SPI1 pins are:

| SPI Signal | Pin |
| --- | --- |
| SCK | PA5 |
| MISO | PA6 |
| MOSI | PA7 |
| NSS | PA4 |

You cannot do:

SCK  -> PC13
MOSI -> PB1
MISO -> PA2

because the SPI1 peripheral is not connected to those pins.

# Example: I2C1

Common I2C1 pins:

| I2C Signal | Pin |
| --- | --- |
| SCL | PB6 |
| SDA | PB7 |

or alternate mappings depending on the MCU.

# How Does STM32 Know?

STM32 uses Alternate Function Registers.

For example:

GPIOA->MODER |= ...
GPIOA->AFR[0] |= ...

When you select:

AF5

on PA5, PA6, PA7

the pins become:

PA5 -> SPI1_SCK
PA6 -> SPI1_MISO
PA7 -> SPI1_MOSI

instead of normal GPIOs.

# Then Why Do Some People Use Any GPIO?

Because they are doing Software SPI (Bit-Banging).

Example:

PB0 -> SCK
PB1 -> MOSI
PB2 -> MISO

Now your code manually toggles pins:

PB0 = 1;
PB0 = 0;

to generate the clock.

This is called:

Software SPI
Bit-Banged SPI

# Hardware SPI vs Software SPI

### Hardware SPI

Uses SPI peripheral
Uses specific AF pins
Very fast
Low CPU usage

Example:

PA5 -> SCK
PA6 -> MISO
PA7 -> MOSI

### Software SPI

Uses any GPIO
CPU generates clock manually
Slower
Higher CPU usage

Example:

PB0 -> SCK
PB1 -> MOSI
PB2 -> MISO

# Same for I2C

### Hardware I2C

Uses dedicated I2C pins.

Example:

PB6 -> SCL
PB7 -> SDA

### Software I2C

Any GPIO can be used.

Example:

PC0 -> SCL
PC1 -> SDA

Your code manually creates:

START
STOP
ACK
NACK

signals.

# For STM32F401 Learning

Always start with:

Hardware SPI
Hardware I2C

using the pins listed in the datasheet/reference manual.

Only learn software SPI/I2C later.

### Interview Answer

Can SPI/I2C use any GPIO pin?

- Hardware SPI/I2C: No. Only pins that support the required Alternate Function can be used.

- Software (bit-banged) SPI/I2C: Yes. Any GPIO pins can be used because the protocol is implemented entirely in software.

Top of Form

XPLAIN WHY WE USE SPI_cr2 registers

Excellent question.

Many beginners learn SPI_CR1 and think that's enough. But SPI_CR2 is used for advanced features that SPI_CR1 does not control.

# SPI Registers Overview

SPI_CR1  --> Basic SPI Configuration

SPI_CR2  --> Advanced SPI Features

SPI_SR   --> Status

SPI_DR   --> Data

# What is SPI_CR2?

SPI_CR2 = SPI Control Register 2

Used for:

- Interrupt control

- DMA control

- NSS management

- Data frame enhancements

# Why Not Put Everything in CR1?

Think like this:

SPI_CR1
---------
Master/Slave
CPOL
CPHA
Baud Rate
Enable SPI

SPI_CR2
---------
Interrupts
DMA
NSS Output
Advanced Features

To keep the design organized, STM32 separates basic and advanced controls.

# Important SPI_CR2 Bits

## 1. TXEIE

Transmit Buffer Empty Interrupt Enable

Bit:

TXEIE

Without Interrupt

CPU continuously checks:

while(!(SPI1->SR & TXE));

This is called polling.

With Interrupt

SPI1->CR2 |= (1<<7);

When TXE becomes 1:

SPI Hardware
      |
      v
Generates Interrupt

CPU automatically enters ISR.

### Why Use It?

Instead of:

CPU checking
CPU checking
CPU checking
CPU checking

SPI tells CPU:

"Hey! My transmit buffer is empty."

More efficient.

# 2. RXNEIE

Receive Buffer Not Empty Interrupt Enable

SPI1->CR2 |= (1<<6);

When data arrives:

RXNE = 1

Interrupt generated.

### Why?

Without interrupt:

while(!(SPI1->SR & RXNE));

CPU wastes time waiting.

With interrupt:

Receive Data
      |
      v
Interrupt
      |
      v
Read SPI_DR

# 3. ERRIE

Error Interrupt Enable

Used to detect:

Overrun Error
Mode Fault
CRC Error

Example:

Slave sends data.

CPU doesn't read DR.

New data arrives.

Overrun Error

ERRIE can generate an interrupt.

# 4. TXDMAEN

Transmit DMA Enable

SPI1->CR2 |= (1<<1);

This is very important.

Without DMA

CPU sends every byte.

SPI1->DR = data1;
SPI1->DR = data2;
SPI1->DR = data3;
...

With DMA

Memory
   |
 DMA
   |
 SPI_DR

DMA automatically fills SPI_DR.

CPU does almost nothing.

### Why Use DMA?

Imagine TFT display:

320 x 240 pixels

Thousands of bytes.

CPU sending each byte is slow.

DMA is much faster.

# 5. RXDMAEN

Receive DMA Enable

SPI1->CR2 |= (1<<0);

DMA automatically copies:

SPI_DR
   |
 DMA
   |
 RAM

No CPU involvement.

# 6. SSOE

Slave Select Output Enable

Very important in Master mode.

SPI1->CR2 |= (1<<2);

Without SSOE

You manually control NSS:

GPIOA->BSRR = ...

With SSOE

SPI hardware automatically controls NSS.

Start Transfer
    |
NSS LOW

End Transfer
    |
NSS HIGH

# Practical Example

Suppose STM32 communicates with an SPI Flash.

### Basic Setup

SPI_CR1:

Master Mode
CPOL
CPHA
Baud Rate
SPI Enable

### Advanced Setup

SPI_CR2:

TXE Interrupt
RXNE Interrupt
DMA
NSS Output

# Real-Life Analogy

Think of a factory.

### SPI_CR1

Factory configuration:

How many workers?
Factory speed?
Factory mode?

### SPI_CR2

Automation features:

Alarm system
Automatic conveyor belt
Notifications
Robots

# Interview Answer

### Why do we use SPI_CR2?

SPI_CR2 provides advanced SPI control features that are not part of the basic SPI configuration.

It is mainly used for:

- TXE interrupts (TXEIE)

- RXNE interrupts (RXNEIE)

- Error interrupts (ERRIE)

- DMA transmit (TXDMAEN)

- DMA receive (RXDMAEN)

- Hardware NSS output (SSOE)

While SPI_CR1 configures how SPI works, SPI_CR2 configures how data transfer is managed efficiently using interrupts and DMA.

Bottom of Form