# Program 1: Polling RX + Polling TX (Character Echo)

### Functionality

Tera Term ---> STM32
                  ↓
             Receive Char
                  ↓
             Send Same Char
                  ↓
Tera Term <--- STM32

### Example

Input:

A

Output:

A

Input:

HELLO

Output:

HELLO

(one character echoed at a time)

## Full Code

#include <stdint.h>

/* Base Addresses */
#define RCC_BASE        0x40023800UL
#define GPIOA_BASE      0x40020000UL
#define USART1_BASE     0x40011000UL

/* RCC Registers */
#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))
#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIOA Registers */
#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))
#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART1 Registers */
#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))
#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))
#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))
#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

void GPIOA_Init(void)
{
    volatile uint32_t delay;

    RCC_AHB1ENR |= (1 << 0);

    delay = RCC_AHB1ENR;
    (void)delay;

    GPIOA_MODER &= ~((3 << 18) | (3 << 20));
    GPIOA_MODER |=  ((2 << 18) | (2 << 20));

    GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));
    GPIOA_AFRH |=  ((7 << 4) | (7 << 8));
}

void USART1_Init(void)
{
    volatile uint32_t delay;

    RCC_APB2ENR |= (1 << 4);

    delay = RCC_APB2ENR;
    (void)delay;

    USART1_BRR = 0x0683;

    USART1_CR1 = (1 << 13) | (1 << 3) | (1 << 2);
}

void USART1_TxChar(char ch)
{
    while (!(USART1_SR & (1 << 7)));

    USART1_DR = ch;
}

char USART1_RxChar(void)
{
    while (!(USART1_SR & (1 << 5)));

    return (char)(USART1_DR & 0xFF);
}

void USART1_TxString(char *str)
{
    while (*str)
    {
        USART1_TxChar(*str++);
    }
}

char data_byte;

int main(void)
{
    GPIOA_Init();

    USART1_Init();

    USART1_TxString("UART Echo Ready\r\n");

    while (1)
    {
        data_byte = USART1_RxChar();

        USART1_TxChar(data_byte);
    }
}

# Program 2: Polling RX + Polling TX (String Echo)

### Functionality

Type complete string
        ↓
Press Enter
        ↓
STM32 stores string
        ↓
STM32 sends same string back

### Example

Input:

HELLO

Press Enter.

Output:

Received: HELLO

## Full Code

#include <stdint.h>

/* Base Addresses */
#define RCC_BASE        0x40023800UL
#define GPIOA_BASE      0x40020000UL
#define USART1_BASE     0x40011000UL

/* RCC Registers */
#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))
#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIO Registers */
#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))
#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART Registers */
#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))
#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))
#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))
#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

char rx_buffer[50];

void GPIOA_Init(void)
{
    volatile uint32_t delay;

    RCC_AHB1ENR |= (1 << 0);

    delay = RCC_AHB1ENR;
    (void)delay;

    GPIOA_MODER &= ~((3 << 18) | (3 << 20));
    GPIOA_MODER |=  ((2 << 18) | (2 << 20));

    GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));
    GPIOA_AFRH |=  ((7 << 4) | (7 << 8));
}

void USART1_Init(void)
{
    volatile uint32_t delay;

    RCC_APB2ENR |= (1 << 4);

    delay = RCC_APB2ENR;
    (void)delay;

    USART1_BRR = 0x0683;

    USART1_CR1 = (1 << 13) | (1 << 3) | (1 << 2);
}

void USART1_TxChar(char ch)
{
    while (!(USART1_SR & (1 << 7)));

    USART1_DR = ch;
}

char USART1_RxChar(void)
{
    while (!(USART1_SR & (1 << 5)));

    return (char)(USART1_DR & 0xFF);
}

void USART1_TxString(char *str)
{
    while (*str)
    {
        USART1_TxChar(*str++);
    }
}

int main(void)
{
    int i;

    GPIOA_Init();

    USART1_Init();

    USART1_TxString("Enter String: ");

    while (1)
    {
        i = 0;

        while (1)
        {
            rx_buffer[i] = USART1_RxChar();

            if (rx_buffer[i] == '\r')
            {
                break;
            }

            i++;
        }

        rx_buffer[i] = '\0';

        USART1_TxString("\r\nReceived: ");
        USART1_TxString(rx_buffer);
        USART1_TxString("\r\n");
    }
}

Full duplex communicatin… continuous data transmission and interrupt reciving code below

#include <stdint.h>

/* Base Addresses */

#define RCC_BASE        0x40023800UL

#define GPIOA_BASE      0x40020000UL

#define USART1_BASE     0x40011000UL

/* RCC Registers */

#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))

#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIOA Registers */

#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))

#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART1 Registers */

#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))

#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))

#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))

#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

/* NVIC Register */

#define NVIC_ISER1      (*(volatile uint32_t *)0xE000E104)

/* Global Variable */

volatile char rx_data;

/* GPIO Initialization */

void GPIOA_Init(void)

{

volatile uint32_t delay;

RCC_AHB1ENR |= (1 << 0);

delay = RCC_AHB1ENR;

(void)delay;

/* PA9 = TX, PA10 = RX */

GPIOA_MODER &= ~((3 << 18) | (3 << 20));

GPIOA_MODER |=  ((2 << 18) | (2 << 20));

GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));

GPIOA_AFRH |=  ((7 << 4) | (7 << 8));

}

/* USART1 Initialization */

void USART1_Init(void)

{

volatile uint32_t delay;

RCC_APB2ENR |= (1 << 4);

delay = RCC_APB2ENR;

(void)delay;

/* 9600 baud @16MHz */

USART1_BRR = 0x0683;

USART1_CR1 =

(1 << 13)      /* UE */

| (1 << 3)       /* TE */

| (1 << 2)       /* RE */

| (1 << 5);      /* RXNEIE */

}

/* Transmit One Character */

void USART1_TxChar(char ch)

{

while (!(USART1_SR & (1 << 7)));

USART1_DR = ch;

}

/* Transmit String */

void USART1_TxString(char *str)

{

while (*str)

{

USART1_TxChar(*str++);

}

}

/* Simple Delay */

void Delay(void)

{

volatile uint32_t i;

for (i = 0; i < 3000000; i++)

{

}

}

/* USART1 Interrupt Handler */

void USART1_IRQHandler(void)

{

if (USART1_SR & (1 << 5))      /* RXNE */

{

rx_data = (char)(USART1_DR & 0xFF);

/* Echo back received character */

while (!(USART1_SR & (1 << 7)));

USART1_DR = rx_data;

}

}

/* Main */

int main(void)

{

GPIOA_Init();

USART1_Init();

/* Enable USART1 Interrupt

USART1 IRQ Number = 37

ISER1 bit = 37 - 32 = 5

*/

NVIC_ISER1 |= (1 << 5);

USART1_TxString("USART1 Full Duplex Ready\r\n");

while (1)

{

USART1_TxString("STM32 Running...\r\n");

Delay();

}

}

Data recivibg when send the data from tera term(interrupt reciving )
and tx polling

#include <stdint.h>

/* Base Addresses */

#define RCC_BASE        0x40023800UL

#define GPIOA_BASE      0x40020000UL

#define USART1_BASE     0x40011000UL

/* RCC Registers */

#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))

#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIO Registers */

#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))

#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART1 Registers */

#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))

#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))

#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))

#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

/* NVIC */

#define NVIC_ISER1      (*(volatile uint32_t *)0xE000E104)

/* Global Variables */

volatile char rx_data;

volatile uint8_t rx_flag = 0;

/* GPIO Init */

void GPIOA_Init(void)

{

volatile uint32_t temp;

RCC_AHB1ENR |= (1 << 0);

temp = RCC_AHB1ENR;

(void)temp;

/* PA9 TX, PA10 RX */

GPIOA_MODER &= ~((3 << 18) | (3 << 20));

GPIOA_MODER |=  ((2 << 18) | (2 << 20));

GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));

GPIOA_AFRH |=  ((7 << 4) | (7 << 8));

}

/* USART1 Init */

void USART1_Init(void)

{

volatile uint32_t temp;

RCC_APB2ENR |= (1 << 4);

temp = RCC_APB2ENR;

(void)temp;

USART1_BRR = 0x0683;      /* 9600 baud @16MHz */

USART1_CR1 =

(1 << 13)    /* UE */

| (1 << 3)     /* TE */

| (1 << 2)     /* RE */

| (1 << 5);    /* RXNEIE */

}

/* TX Polling */

void USART1_TxChar(char ch)

{

while (!(USART1_SR & (1 << 7)));

USART1_DR = ch;

}

/* String TX */

void USART1_TxString(char *str)

{

while (*str)

{

USART1_TxChar(*str++);

}

}

/* USART1 Interrupt Handler */

void USART1_IRQHandler(void)

{

if (USART1_SR & (1 << 5))     /* RXNE */

{

rx_data = (char)(USART1_DR & 0xFF);

rx_flag = 1;

}

}

int main(void)

{

GPIOA_Init();

USART1_Init();

/* USART1 IRQ = 37

ISER1 bit = 37-32 = 5

*/

NVIC_ISER1 |= (1 << 5);

USART1_TxString("RX Interrupt Demo Ready\r\n");

while (1)

{

if (rx_flag)

{

rx_flag = 0;

/* TX using polling */

USART1_TxChar(rx_data);

}

}

}

Uart Tx_interrupt  data

#include <stdint.h>

/* Base Addresses */

#define RCC_BASE        0x40023800UL

#define GPIOA_BASE      0x40020000UL

#define USART1_BASE     0x40011000UL

/* RCC Registers */

#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))

#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIO Registers */

#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))

#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART Registers */

#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))

#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))

#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))

#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

/* NVIC */

#define NVIC_ISER1      (*(volatile uint32_t *)0xE000E104)

/* Message to transmit */

char tx_msg[] = "HELLO FROM STM32\r\n";

volatile uint32_t tx_index = 0;

/* GPIO Init */

void GPIOA_Init(void)

{

volatile uint32_t temp;

RCC_AHB1ENR |= (1 << 0);

temp = RCC_AHB1ENR;

(void)temp;

GPIOA_MODER &= ~((3 << 18) | (3 << 20));

GPIOA_MODER |=  ((2 << 18) | (2 << 20));

GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));

GPIOA_AFRH |=  ((7 << 4) | (7 << 8));

}

/* USART Init */

void USART1_Init(void)

{

volatile uint32_t temp;

RCC_APB2ENR |= (1 << 4);

temp = RCC_APB2ENR;

(void)temp;

USART1_BRR = 0x0683;

USART1_CR1 =

(1 << 13)     /* UE */

| (1 << 3);     /* TE */

}

/* USART1 Interrupt Handler */

void USART1_IRQHandler(void)

{

/* TXE Interrupt */

if (USART1_SR & (1 << 7))

{

if (tx_msg[tx_index] != '\0')

{

USART1_DR = tx_msg[tx_index++];

}

else

{

/* Disable TXE interrupt */

USART1_CR1 &= ~(1 << 7);

tx_index = 0;

}

}

}

int main(void)

{

GPIOA_Init();

USART1_Init();

/* Enable USART1 IRQ (IRQ37) */

NVIC_ISER1 |= (1 << 5);

/* Enable TXE Interrupt */

USART1_CR1 |= (1 << 7);

while (1)

{

}

}