1_STM32F401x_Datasheet.pdf

PDF

Create concise professional notes for: "Introduction to STM32F401" Cover only: - What is STM32 - STM32F401 overview - Cortex-M4 overview - Flash and SRAM - Memory map basics - GPIO overview - Clock overview - Peripheral overview - STM32F401 block diagram - Development tools - STM32CubeIDE setup - ST-Link setup - Build process basics - First LED blink project Include: - important diagrams - STM32 architecture diagram - memory map diagram - complete LED blink bare-metal code - code explanation explain in detail these all things in detail with regester level

# Introduction to STM32F401 Microcontroller

The STM32F401 belongs to the STM32F4 family from STMicroelectronics.
It is a high-performance 32-bit microcontroller based on the ARM Cortex-M4 core with FPU.

# 1. What is STM32?

STM32 is a family of 32-bit microcontrollers developed by STMicroelectronics based on ARM Cortex cores.

## Main Features

- 32-bit ARM architecture

- High speed processing

- Low power consumption

- Rich peripherals

- Flash memory + SRAM

- GPIO, Timers, UART, SPI, I2C, ADC, USB etc.

# STM32 Family Overview

| Family | Core | Usage |
| --- | --- | --- |
| STM32F0 | Cortex-M0 | Basic low-cost |
| STM32F1 | Cortex-M3 | General purpose |
| STM32F4 | Cortex-M4 | High performance |
| STM32H7 | Cortex-M7 | Very high speed |
| STM32L4 | Cortex-M4 | Low power |

# 2. STM32F401 Overview

The STM32F401 is a high-performance MCU with:

| Feature | Value |
| --- | --- |
| Core | ARM Cortex-M4 |
| Max Clock | 84 MHz |
| Flash | 256 KB |
| SRAM | 64 KB |
| GPIO | Up to 81 |
| ADC | 12-bit |
| Timers | 11 |
| Interfaces | UART, SPI, I2C, USB |

# STM32F401 Internal Architecture

+----------------------+
                |   Cortex-M4 Core     |
                |      CPU + FPU       |
                +----------+-----------+
                           |
          ---------------------------------------
          |                 |                   |
      Flash Memory       SRAM             NVIC
          |                 |                   |
          ---------------------------------------
                           |
                    AHB/APB BUS
                           |
 ---------------------------------------------------------
 |        |         |        |        |        |         |
GPIO    UART      SPI      I2C      ADC     TIMERS     USB

# 3. Cortex-M4 Overview

The STM32F401 uses the ARM Cortex-M4 processor.

## Features

- 32-bit RISC CPU

- DSP Instructions

- Floating Point Unit (FPU)

- Nested Interrupt Controller (NVIC)

- SysTick Timer

# Cortex-M4 Architecture

+----------------------+
          |     Cortex-M4 CPU    |
          +----------------------+
          | Registers            |
          | ALU                  |
          | FPU                  |
          | DSP Engine           |
          | NVIC                 |
          | SysTick              |
          +----------------------+

# ARM Register Set

The Cortex-M4 contains:

| Register | Purpose |
| --- | --- |
| R0-R12 | General registers |
| SP | Stack Pointer |
| LR | Link Register |
| PC | Program Counter |
| xPSR | Status Register |

# Example

MOV R0,#10
MOV R1,#20
ADD R2,R0,R1

This adds 10 + 20.

# 4. Flash and SRAM

## Flash Memory

Used to store:

- Program code

- Constants

- Firmware

STM32F401 Flash starts at:

0x08000000

Size:

256 KB

## SRAM

Used for:

- Variables

- Stack

- Heap

SRAM starts at:

0x20000000

Size:

64 KB

# Memory Architecture Diagram

FLASH MEMORY
+-------------------+
| 0x08000000        |
|                   |
|   Program Code    |
|                   |
+-------------------+

  SRAM
+-------------------+
| 0x20000000        |
|                   |
| Variables/Stack   |
|                   |
+-------------------+

# 5. Memory Map Basics

STM32 uses memory mapped I/O.

That means peripherals are accessed like memory addresses.

# STM32 Memory Map

Address Range            Purpose
------------------------------------------------
0x08000000 - Flash
0x20000000 - SRAM
0x40000000 - Peripherals
0xE0000000 - Cortex-M4 Internal

# Peripheral Base Addresses

| Peripheral | Address |
| --- | --- |
| GPIOA | 0x40020000 |
| GPIOB | 0x40020400 |
| RCC | 0x40023800 |

# Register-Level Access

Example:

*(volatile unsigned int*)0x40023830 |= (1<<0);

This enables GPIOA clock.

# 6. GPIO Overview

GPIO = General Purpose Input Output.

Used for:

- LED control

- Button reading

- Digital communication

# GPIO Block Diagram

+----------------+
PIN ----> | Input Buffer   |
          | Output Driver  |
          | Pull-up/down   |
          | Alternate Func |
          +----------------+

# GPIO Registers

| Register | Purpose |
| --- | --- |
| MODER | Mode select |
| OTYPER | Output type |
| OSPEEDR | Speed |
| PUPDR | Pull control |
| IDR | Input data |
| ODR | Output data |
| BSRR | Bit set/reset |

# GPIO Modes

| Mode | Binary |
| --- | --- |
| Input | 00 |
| Output | 01 |
| Alternate | 10 |
| Analog | 11 |

# GPIO Register-Level Example

## Enable GPIOA Clock

RCC->AHB1ENR |= (1<<0);

Equivalent register access:

*(volatile unsigned int*)0x40023830 |= (1<<0);

## Configure PA5 Output

GPIOA->MODER &= ~(3<<10);
GPIOA->MODER |=  (1<<10);

Explanation:

PA5 uses bits 10 and 11.

01 = Output Mode

# 7. Clock Overview

Clock controls CPU and peripherals.

Without clock:

- peripheral does NOT work

# Clock Sources

| Clock | Description |
| --- | --- |
| HSI | Internal 16 MHz |
| HSE | External crystal |
| PLL | Multiplies frequency |
| LSI | Low-speed internal |
| LSE | RTC crystal |

# Clock Tree

HSI/HSE
                |
               PLL
                |
         +------+------+
         |             |
       AHB           APB
         |             |
      GPIO         UART/SPI

# Important Clock Registers

| Register | Purpose |
| --- | --- |
| RCC_CR | Clock control |
| RCC_PLLCFGR | PLL config |
| RCC_CFGR | Clock select |
| RCC_AHB1ENR | GPIO clocks |

# Example: Enable GPIOA Clock

RCC->AHB1ENR |= (1<<0);

Bit 0 enables GPIOA.

# 8. Peripheral Overview

STM32F401 has many peripherals.

# Communication Peripherals

| Peripheral | Use |
| --- | --- |
| UART | Serial communication |
| SPI | High-speed communication |
| I2C | Sensor communication |
| USB | USB interface |

# Timers

Used for:

- Delays

- PWM

- Motor control

- Frequency measurement

# ADC

Converts analog voltage to digital value.

0V → 0
3.3V → 4095

12-bit ADC:

2^12 = 4096 levels

# DMA

DMA transfers data without CPU involvement.

Peripheral <-> Memory

# 9. STM32F401 Block Diagram

+----------------+
                | Cortex-M4 CPU  |
                +----------------+
                         |
              +----------+----------+
              |                     |
           Flash                  SRAM
              |                     |
              +----------+----------+
                         |
                   Bus Matrix
                         |
 ------------------------------------------------
 | GPIO | UART | SPI | I2C | ADC | DMA | TIMERS |
 ------------------------------------------------

# 10. Development Tools

| Tool | Purpose |
| --- | --- |
| STM32CubeIDE | Coding/debugging |
| ST-Link | Programming |
| CubeMX | Peripheral config |
| OpenOCD | Debugging |

# 11. STM32CubeIDE Setup

Download from:

STM32CubeIDE Download

# Installation Steps

- Install IDE

- Install STM32 drivers

- Connect board

- Create project

- Select STM32F401

- Build project

- Flash MCU

# 12. ST-Link Setup

ST-Link is used for:

- Flashing

- Debugging

- Breakpoints

# ST-Link Connections

| ST-Link | STM32 |
| --- | --- |
| SWDIO | SWDIO |
| SWCLK | SWCLK |
| GND | GND |
| 3.3V | VCC |

# 13. Build Process Basics

C Code
  ↓
Preprocessor
  ↓
Compiler
  ↓
Assembler
  ↓
Linker
  ↓
HEX/BIN File
  ↓
Flash into STM32

# GCC Build Commands

arm-none-eabi-gcc
arm-none-eabi-ld
arm-none-eabi-objcopy

# 14. First LED Blink Project (Bare Metal)

# Goal

Blink LED on PA5.

(Common on Nucleo boards)

# Step 1: Enable GPIOA Clock

Register:

RCC_AHB1ENR

Address:

0x40023830

Bit:

Bit0 = GPIOA Enable

# Step 2: Configure PA5 Output

Register:

GPIOA_MODER

Address:

0x40020000

PA5 uses:

MODER5[1:0]

Set:

01 = Output

# Step 3: Toggle Output

Register:

GPIOA_ODR

# Complete Bare-Metal LED Blink Code

#include <stdint.h>

#define RCC_BASE        0x40023800
#define GPIOA_BASE      0x40020000

#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))

#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))
#define GPIOA_ODR       (*(volatile uint32_t *)(GPIOA_BASE + 0x14))

void delay(void)
{
    for(uint32_t i=0; i<500000; i++);
}

int main(void)
{
    /* Enable GPIOA clock */
    RCC_AHB1ENR |= (1 << 0);

    /* Clear MODER5 bits */
    GPIOA_MODER &= ~(3 << 10);

    /* Set PA5 as output */
    GPIOA_MODER |=  (1 << 10);

    while(1)
    {
        /* LED ON */
        GPIOA_ODR |= (1 << 5);

        delay();

        /* LED OFF */
        GPIOA_ODR &= ~(1 << 5);

        delay();
    }
}

# Code Explanation

## Enable Clock

RCC_AHB1ENR |= (1 << 0);

Enables GPIOA peripheral clock.

Without this GPIO registers will not work.

## Configure PA5

GPIOA_MODER &= ~(3 << 10);
GPIOA_MODER |=  (1 << 10);

### Why 10?

Each GPIO pin uses 2 bits.

For pin 5:

5 × 2 = 10

Bits 11:10 belong to PA5.

# MODER Calculation

00 = Input
01 = Output
10 = Alternate
11 = Analog

We write:

01

So PA5 becomes output.

## Turn LED ON

GPIOA_ODR |= (1 << 5);

Sets bit 5.

PA5 becomes HIGH.

## Turn LED OFF

GPIOA_ODR &= ~(1 << 5);

Clears bit 5.

PA5 becomes LOW.

# Register-Level Summary

| Register | Address | Purpose |
| --- | --- | --- |
| RCC_AHB1ENR | 0x40023830 | Enable GPIO clock |
| GPIOA_MODER | 0x40020000 | Configure mode |
| GPIOA_ODR | 0x40020014 | Output control |

# Execution Flow

CPU Reset
   ↓
main()
   ↓
Enable GPIO Clock
   ↓
Configure PA5 Output
   ↓
Set PA5 HIGH
   ↓
Delay
   ↓
Set PA5 LOW
   ↓
Repeat Forever

# Important Concepts Learned

- ARM Cortex-M4

- Register-level programming

- Memory mapped I/O

- GPIO control

- Clock enabling

- Bare-metal embedded programming

- STM32 architecture

# Important Notes

- Every peripheral needs clock enable first

- Registers are memory addresses

- GPIO mode must be configured

- Bare-metal means NO HAL/library

# References

- STM32F401 Datasheet

- RM0368 Reference Manual

- Cortex-M4 Programming Manual PM0214

4\. Flash and SRAM ================== Flash Memory ------------ Used to store: * Program code * Constants * Firmware STM32F401 Flash starts at: 0x08000000 Size: 256 KB * * * SRAM ---- Used for: * Variables * Stack * Heap SRAM starts at: 0x20000000 Size: 64 KB 1\_STM32F401x\_Datasheet * * * Memory Architecture Diagram =========================== FLASH MEMORY+-------------------+| 0x08000000 || || Program Code || |+-------------------+ SRAM+-------------------+| 0x20000000 || || Variables/Stack || |+-------------------+

explain in detail

# Flash and SRAM in STM32F401 Microcontroller — Detailed Explanation

In STM32 microcontrollers, memory is mainly divided into:

- Flash Memory

- SRAM (Static RAM)

These are directly connected to the ARM Cortex-M4 core through internal buses.

# Basic Difference

| Memory | Purpose | Data Retained After Power OFF? |
| --- | --- | --- |
| Flash | Store program permanently | YES |
| SRAM | Store temporary runtime data | NO |

# Simple Understanding

Think like this:

| Real Life Example | STM32 Memory |
| --- | --- |
| Hard Disk / SSD | Flash |
| Computer RAM | SRAM |

# 1. Flash Memory

# What is Flash?

Flash memory is non-volatile memory.

That means:

Power OFF → Data remains saved

It stores:

- Program code

- Firmware

- Constants

- Lookup tables

- Interrupt vector table

# STM32F401 Flash Details

| Parameter | Value |
| --- | --- |
| Start Address | 0x08000000 |
| Size | 256 KB |
| Type | Embedded Flash |
| Access | Read/Write |

# Flash Memory Layout

FLASH MEMORY
Address Range

0x08000000  +------------------+
             | Vector Table     |
             +------------------+
             | Program Code     |
             |                  |
             | Functions        |
             | Loops            |
             | Logic            |
             +------------------+
             | Constants        |
             | Strings          |
             +------------------+

0x08040000  End of 256 KB Flash

# Why Flash Starts at 0x08000000?

ARM Cortex-M architecture defines memory regions.

STM32 maps internal Flash at:

0x08000000

So CPU fetches instructions from there after reset.

# Program Execution Flow

When MCU powers ON:

CPU RESET
   ↓
PC loads address from vector table
   ↓
Execution starts from FLASH

# Vector Table

The first few bytes of Flash contain:

| Address | Purpose |
| --- | --- |
| 0x08000000 | Initial Stack Pointer |
| 0x08000004 | Reset Handler Address |

# Example

Suppose Flash contains:

0x08000000 → 0x20010000
0x08000004 → 0x08000121

Meaning:

- Stack starts at SRAM address

- Program starts at Reset_Handler()

# Flash Sectors

STM32F401 Flash is divided into sectors.

## Why sectors?

Because Flash erase works sector-wise.

# Sector Example

| Sector | Size |
| --- | --- |
| Sector 0 | 16 KB |
| Sector 1 | 16 KB |
| Sector 2 | 16 KB |
| Sector 3 | 16 KB |
| Sector 4 | 64 KB |
| Sector 5 | 128 KB |

# Flash Operations

Flash supports:

| Operation | Meaning |
| --- | --- |
| Read | Read data |
| Program | Write data |
| Erase | Clear sector |

# Important Flash Rule

You CANNOT directly overwrite Flash.

Steps:

1. Erase sector
2. Program new data

# Flash Register-Level Access

Flash controller registers are located in:

0x40023C00

# Important Flash Registers

| Register | Purpose |
| --- | --- |
| FLASH_ACR | Access control |
| FLASH_KEYR | Unlock Flash |
| FLASH_SR | Status |
| FLASH_CR | Control |

# FLASH_ACR Register

Controls:

- Wait states

- Prefetch

- Cache

# Why Wait States?

Flash is slower than CPU.

CPU:

84 MHz

Flash cannot always respond instantly.

So wait states are inserted.

# Example

At high frequency:

CPU requests instruction
     ↓
Flash needs extra time
     ↓
Wait state inserted

# ART Accelerator

STM32F401 includes:

ART Accelerator

This improves Flash execution speed.

It provides:

- Instruction prefetch

- Branch cache

So code runs nearly at:

0 wait state performance

# Flash Programming Example

#define FLASH_ACR (*(volatile unsigned int*)0x40023C00)

FLASH_ACR |= (1<<8);

Enables prefetch.

# 2. SRAM

# What is SRAM?

SRAM = Static Random Access Memory.

This is volatile memory.

Meaning:

Power OFF → Data lost

# SRAM Purpose

Used during program execution.

Stores:

- Variables

- Stack

- Heap

- Function parameters

- Buffers

# STM32F401 SRAM Details

| Parameter | Value |
| --- | --- |
| Start Address | 0x20000000 |
| Size | 64 KB |
| Access Speed | Very Fast |
| Wait States | 0 |

# SRAM Layout

SRAM
0x20000000  +------------------+
             | Global Variables |
             +------------------+
             | Static Variables |
             +------------------+
             | Heap             |
             | grows upward ↑   |
             +------------------+
             |                  |
             | Free Space       |
             |                  |
             +------------------+
             | Stack            |
             | grows downward ↓ |
             +------------------+

0x20010000  End of SRAM

# Stack in SRAM

Stack stores:

- Function calls

- Local variables

- Return addresses

# Stack Example

void fun()
{
   int x = 10;
}

Variable:

x

Stored in stack.

# Stack Growth

ARM stack grows downward.

Example:

Before push → 0x20010000
After push  → 0x2000FFFC

# Heap in SRAM

Heap used for dynamic memory.

Functions:

malloc()
free()

# Global Variables

Example:

int a = 10;

Stored in SRAM.

# Static Variables

static int count;

Also stored in SRAM.

# Local Variables

void fun()
{
   int x;
}

Stored in stack area of SRAM.

# Register-Level SRAM Access

You can directly access SRAM.

Example:

volatile unsigned int *ptr;

ptr = (unsigned int*)0x20000000;

*ptr = 0x12345678;

# What Happens Internally?

CPU writes data
      ↓
AHB Bus
      ↓
SRAM Controller
      ↓
Data stored in SRAM cell

# SRAM Access Speed

SRAM is much faster than Flash.

Why?

Because SRAM does NOT require erase/program cycles.

# Flash vs SRAM Speed

| Feature | Flash | SRAM |
| --- | --- | --- |
| Speed | Slower | Faster |
| Persistent | YES | NO |
| Used For | Program | Variables |
| Wait States | Sometimes | No |

# CPU and Memory Connection

+----------------+
                | Cortex-M4 CPU  |
                +--------+-------+
                         |
          -------------------------------
          |                             |
      FLASH                         SRAM
   0x08000000                    0x20000000

# Memory Access Example

Suppose:

int a = 5;

Process:

Variable stored in SRAM

Suppose:

while(1)
{
}

Instructions stored in:

FLASH

# Code Execution Example

int main()
{
    int x = 10;

    while(1)
    {
        x++;
    }
}

# Internally

| Item | Stored In |
| --- | --- |
| main() instructions | Flash |
| variable x | SRAM |
| Stack | SRAM |

# Memory Mapped Architecture

STM32 uses:

Memory Mapped Architecture

Meaning:

Everything has address.

Including:

- Flash

- SRAM

- GPIO

- UART

- Timers

# Complete Memory Map

0x00000000  Boot Memory
0x08000000  Flash
0x20000000  SRAM
0x40000000  Peripherals
0xE0000000  Cortex-M4 Internal

# Why This is Important?

Because embedded programming is mostly:

Reading/Writing addresses

Example:

GPIOA_ODR = 0x20;

Actually writes to a memory address.

# Important Interview Questions

# Why Flash is non-volatile?

Because it stores charge in floating-gate transistors.

# Why SRAM is faster?

Because SRAM uses flip-flop cells.

No erase/program cycle required.

# Why stack grows downward?

ARM architecture design choice for efficient memory usage.

# Why Flash execution needs accelerator?

CPU frequency is higher than Flash access speed.

# Key Points Summary

| Topic | Important Point |
| --- | --- |
| Flash | Stores program permanently |
| SRAM | Stores runtime data |
| Flash Start | 0x08000000 |
| SRAM Start | 0x20000000 |
| Flash Size | 256 KB |
| SRAM Size | 64 KB |
| Stack | In SRAM |
| Variables | In SRAM |
| Code | In Flash |
| Flash slower? | YES |
| SRAM faster? | YES |

Raayanmini_v4.0_4.1_Schematic.pdf

PDF

1_STM32F401x_Datasheet.pdf

PDF

2_STM32F401x_Ref_Manual (1).pdf

PDF

CortexM4_STM32_Programming_Manual.pdf

PDF

these just give codes acoordily this boARD OKAY

Because your switch interrupt uses:

EXTI9

and in STM32:

EXTI lines 5 to 9 share one NVIC interrupt

called:

EXTI9_5

# VERY IMPORTANT CONCEPT

There are:

16 EXTI lines

but NOT all have separate NVIC interrupt numbers.

STM32 groups some EXTI lines together.

# EXTI Grouping

| EXTI Line | NVIC IRQ Name | IRQ Number |
| --- | --- | --- |
| EXTI0 | EXTI0_IRQn | 6 |
| EXTI1 | EXTI1_IRQn | 7 |
| EXTI2 | EXTI2_IRQn | 8 |
| EXTI3 | EXTI3_IRQn | 9 |
| EXTI4 | EXTI4_IRQn | 10 |
| EXTI5-9 | EXTI9_5_IRQn | 23 |
| EXTI10-15 | EXTI15_10_IRQn | 40 |

# Your Case

You are using:

PC9

which connects to:

EXTI9

# EXTI9 Belongs To Group

EXTI5-9

# Therefore NVIC Interrupt Is

EXTI9_5_IRQn

whose IRQ number is:

23

# Therefore We Enable

NVIC_ISER0 |= (1<<23);

# Meaning

Enable IRQ23 in NVIC

which means:

Allow EXTI5-9 interrupts to reach CPU

# Internal Flow

PC9 button pressed
       ↓
EXTI9 detects edge
       ↓
EXTI9 belongs to EXTI5_9 group
       ↓
NVIC generates IRQ23
       ↓
CPU executes ISR

# Why Grouping?

To save hardware resources.

Instead of giving separate IRQ for every EXTI line:

STM32 groups multiple EXTI lines together

# Then How CPU Knows Which Pin Triggered?

Inside ISR we check:

EXTI_PR register

# Example

if(EXTI_PR & (1<<9))
{
    EXTI_PR |= (1<<9);

    GPIOB_ODR ^= (1<<13);
}

# Meaning

Check whether EXTI9 caused interrupt

# Visual Understanding

EXTI5
EXTI6
EXTI7
EXTI8
EXTI9
   ↓
Grouped Together
   ↓
IRQ23
   ↓
NVIC
   ↓
CPU

# Why ISER0 ?

NVIC interrupt enable registers:

| Register | Controls IRQ |
| --- | --- |
| ISER0 | IRQ0-31 |
| ISER1 | IRQ32-63 |

Since:

IRQ23 < 32

we use:

NVIC_ISER0

# Final Understanding

We enabled:

NVIC_ISER0 |= (1<<23);

because:

PC9 uses EXTI9
EXTI9 belongs to EXTI5-9 group
EXTI5-9 group uses IRQ23

therefore:

IRQ23 must be enabled in NVIC

why we are used here while(1) { } this onw what is use of while here

# Why Do We Use

while(1)
{
}

in embedded systems?

This is one of the MOST important concepts in microcontrollers.

# Simple Answer

To keep the program running forever

# What Happens Without It?

Suppose:

int main(void)
{
    GPIO_Init();

    return 0;
}

# Problem

After executing:

return 0;

program finishes.

But microcontroller firmware should NEVER stop.

# In PCs

When program ends:

Operating System takes control

like Windows/Linux.

# But In STM32

There is NO operating system normally.

So after main() ends:

CPU has nowhere to go

This causes:

- undefined behavior

- crashes

- random execution

# Therefore

Embedded programs usually run:

FOREVER

# Infinite Loop

while(1)
{
}

means:

Loop forever

because:

1 = always TRUE

# CPU Behavior

while(1)
{
}

becomes:

Check condition → TRUE
Run loop
Go back
Check condition → TRUE
Run loop
...
forever

# Why Important In Interrupt Programs?

Example:

while(1)
{
}

with interrupt system enabled.

# Actual Behavior

CPU waits here
       ↓
Interrupt occurs
       ↓
CPU jumps to ISR
       ↓
ISR executes
       ↓
Returns back to while loop

# VERY IMPORTANT UNDERSTANDING

Main loop keeps CPU alive.

Interrupts work in background.

# Visual Flow

main()
{
    setup();

    while(1)
    {
    }
}

# CPU Execution

Setup hardware
      ↓
Enter infinite loop
      ↓
Wait forever
      ↓
Interrupt happens
      ↓
Execute ISR
      ↓
Return to loop

# In Your Interrupt Example

while(1)
{
}

means:

CPU stays alive waiting for button interrupts

# Why Empty Loop?

Because all work happens inside:

ISR (Interrupt Service Routine)

Example:

void EXTI9_5_IRQHandler(void)
{
    GPIOB_ODR ^= (1<<13);
}

# So Main Loop Does Nothing

Interrupt hardware automatically calls ISR.

# Real Embedded Systems

Often loop contains tasks.

Example:

while(1)
{
    read_sensor();
    update_display();
}

Interrupts still work simultaneously.

# Cortex-M4 Internally

While CPU running infinite loop:

NVIC continuously monitors interrupts

# When Interrupt Comes

Hardware automatically:

1. Saves registers
2. Jumps to ISR
3. Executes ISR
4. Restores registers
5. Returns to loop

# Without While Loop

Suppose:

int main()
{
   setup_interrupt();
}

Then:

main() finishes immediately

CPU may execute invalid memory.

# Therefore Embedded Rule

Almost every firmware contains:

while(1)
{
}

or:

for(;;)
{
}

Both mean:

Run forever

# Final Mental Model

Initialization
      ↓
Infinite Loop
      ↓
CPU stays alive
      ↓
Interrupts handled anytime

# MOST IMPORTANT UNDERSTANDING

while(1)
{
}

does NOT create interrupt.

It simply:

Keeps the microcontroller program alive forever

so interrupts and hardware can continue working.

void EXTI9_5_IRQHandler(void) { /* Clear pending interrupt */ EXTI_PR |= (1<<9); /* Toggle PB13 */ GPIOB_ODR ^= (1<<13); } this interput handler name ur wish or is it predefined

# This Name is NOT Your Wish

void EXTI9_5_IRQHandler(void)

is a:

Predefined interrupt handler name

required by ARM/STM32 startup system.

# VERY IMPORTANT

Interrupt handler names are fixed because:

Vector table uses these exact names

# What Happens During Interrupt?

Suppose:

PC9 button pressed

# Flow

EXTI9 interrupt occurs
       ↓
NVIC generates IRQ23
       ↓
CPU checks vector table
       ↓
Finds EXTI9_5_IRQHandler address
       ↓
Jumps to that function

# So Function Name Must Match

If you write wrong name:

void my_interrupt(void)
{
}

CPU will NEVER call it.

# Because Vector Table Expects

EXTI9_5_IRQHandler

exactly.

# Where Is This Defined?

Inside startup file.

Example:

startup_stm32f401xx.s

# Vector Table Example

.word EXTI0_IRQHandler
.word EXTI1_IRQHandler
.word EXTI2_IRQHandler
...
.word EXTI9_5_IRQHandler

# Meaning

Vector table stores:

Address of interrupt functions

# CPU Operation

When IRQ23 occurs:

CPU automatically jumps to:
EXTI9_5_IRQHandler()

# Why Name Contains 9_5?

Because one IRQ handles:

EXTI5
EXTI6
EXTI7
EXTI8
EXTI9

all together.

# Therefore ISR Name Is

EXTI9_5_IRQHandler

# Other Predefined Handler Names

| Interrupt | Handler Name |
| --- | --- |
| EXTI0 | EXTI0_IRQHandler |
| EXTI1 | EXTI1_IRQHandler |
| TIM2 | TIM2_IRQHandler |
| USART1 | USART1_IRQHandler |
| SysTick | SysTick_Handler |

# SysTick Example

void SysTick_Handler(void)
{
}

Again:

Name is predefined

# Important Concept

Interrupt handler names come from:

CMSIS + Startup Vector Table

# Startup File Declares Them Weakly

Example:

void EXTI9_5_IRQHandler(void)
__attribute__((weak, alias("Default_Handler")));

# Meaning

Initially:

All interrupts point to Default_Handler

# When You Create Function

void EXTI9_5_IRQHandler(void)
{
}

your function replaces weak default handler.

# VERY IMPORTANT Embedded Concept

Interrupt execution happens by:

Function address lookup from vector table

NOT by calling function manually.

# Visual Flow

IRQ23 occurs
      ↓
NVIC informs CPU
      ↓
CPU checks vector table
      ↓
Find EXTI9_5_IRQHandler address
      ↓
Jump to ISR

# Final Understanding

void EXTI9_5_IRQHandler(void)

is:

A predefined interrupt handler function name required by STM32 startup/vector table system

and NOT an arbitrary user-chosen name.

Top of Form

Bottom of Form