2_STM32F401x_Ref_Manual (1).pdf

PDF

CortexM4_STM32_Programming_Manual.pdf

PDF

1_STM32F401x_Datasheet.pdf

PDF

Raayanmini_v4.0_4.1_Schematic.pdf

PDF

based on these explain the chapter Create concise professional notes for: just read this pdf and i dont know about anything about ADC explain from scratch to end each thing in detail about adc

Excellent observation.

In your code:

ADC_SQR3 |= 0x0B;

you configured:

Rank1 = CH11

But you never configured:

ADC_SQR1

which contains the L[3:0] field (sequence length).

So why does the code still work?

# Because Reset Value of SQR1 is 0

From the reference manual:

ADC_SQR1 Reset Value = 0x00000000

Therefore:

L[3:0] = 0000

# Meaning of L Field

STM32 defines:

Number of conversions = L + 1

So:

| L Value | Conversions |
| --- | --- |
| 0 | 1 conversion |
| 1 | 2 conversions |
| 2 | 3 conversions |
| ... | ... |
| 15 | 16 conversions |

# In Your Code

Since:

L = 0

ADC assumes:

Number of conversions = 0 + 1
                      = 1

Thus sequence is:

Rank1 = CH11

Only one conversion.

That's exactly what your code wants.

# What If We Wanted 3 Channels?

Suppose:

Rank1 = CH11
Rank2 = CH5
Rank3 = CH2

Then we must tell ADC:

Total conversions = 3

Therefore:

L = 2

because:

L + 1 = 3

Code:

ADC_SQR1 &= ~(0xF << 20);
ADC_SQR1 |=  (2 << 20);

# What Happens If You Forget L?

Suppose you configure:

Rank1 = CH11
Rank2 = CH5
Rank3 = CH2

but leave:

L = 0

ADC will do:

CH11 only

and ignore:

CH5
CH2

because ADC thinks:

Sequence Length = 1

# Why Tutorial Code Doesn't Use SQR1

Because it is a single-channel ADC example.

The programmer relies on the default reset value:

SQR1 = 0
↓
L = 0
↓
1 conversion

So no need to write:

ADC_SQR1 |= (0 << 20);

because it's already zero after reset.

### Easy Memory Trick

For ADC sequence length:

L = Number of Conversions - 1

Examples:

1 channel  → L=0
2 channels → L=1
3 channels → L=2
4 channels → L=3

In your code there is only one channel (CH11), so the default:

L = 0

is already correct, which is why ADC_SQR1 was not used.

ADC_CR2 |= (0x1 << 10); in this program eoc set by software right but why i wrote here in init function mannualy

Yes, exactly.

This line:

ADC_CCR |= (0x1 << 23);

enables the internal temperature sensor inside the STM32.

# What is ADC_CCR?

ADC_CCR means:

ADC Common Control Register

It contains settings shared by ADC peripherals.

One of its bits is:

TSVREFE (Bit 23)

Meaning:

Temperature Sensor and VREFINT Enable

# When You Set Bit 23

ADC_CCR |= (1 << 23);

you are telling STM32:

Connect the internal temperature sensor
to the ADC.

and also:

Enable VREFINT
(Internal Reference Voltage)

# Is Temperature Sensor Built Into STM32?

✅ Yes.

STM32F401 contains an on-chip temperature sensor.

You do not connect any external wire.

Internally it looks like:

Temperature Sensor
        |
        V
     ADC CH16

# ADC Channel Number

The internal temperature sensor is:

ADC Channel 16

Not:

CH10
CH11
PC0
PC1

Those are external pins.

# Internal Channels

STM32 ADC has:

| Channel | Source |
| --- | --- |
| CH0-CH15 | External GPIO Pins |
| CH16 | Internal Temperature Sensor |
| CH17 | VBAT |
| CH18 | VREFINT |

# Small Mistake In Your Program

You enabled the temperature sensor:

ADC_CCR |= (1 << 23);

But your code selects:

ADC_SQR3 |= 0x0A;

which means:

CH10

And:

CH10 = PC0

So your ADC is actually reading:

PC0 voltage

NOT the internal temperature sensor.

# To Read Internal Temperature Sensor

You must select:

CH16

So:

ADC_SQR3 = 16;

or

ADC_SQR3 |= 0x10;

because:

16 decimal = 0x10

Then ADC converts:

Internal Temperature Sensor

instead of PC0.

# Why Formula Uses 0.76V?

STM32 factory calibration says:

At:

25°C

the temperature sensor output is approximately:

0.76V

and slope is approximately:

2.5 mV/°C

Therefore:

Temperature=(Vsense−0.76)0.0025+25Temperature=0.0025(Vsense​−0.76)​+25

which is exactly your formula.

# Visual Diagram

STM32 Chip
--------------------------------

Temperature Sensor
        |
        |
      CH16
        |
        V
      ADC
        |
        V
     ADC_DR

--------------------------------

No external sensor required.

### One-Line Answer

Yes, ADC_CCR |= (1<<23); enables the built-in (on-chip) temperature sensor and VREFINT inside the STM32. To actually read the temperature sensor, the ADC channel must be configured as Channel 16, not Channel 10.

see the diagram explain why in code i am using channel 10 and but enabling internal tem sensor

ok tell me this is contionous mode or single conversion

Your code is Single Conversion Mode.

Reason:

In STM32 ADC:

ADC_CR2 Bit 1 = CONT

| CONT | Mode |
| --- | --- |
| 0 | Single Conversion |
| 1 | Continuous Conversion |

In your code I don't see:

ADC_CR2 |= (1 << 1);

Therefore:

CONT = 0 (default reset value)

which means:

Single Conversion Mode

Your flow is:

ADC_CR2 |= (1 << 30);   // SWSTART

ADC does:

Start Conversion
      ↓
Convert CH10
      ↓
Set EOC
      ↓
Stop

Then next conversion happens only when your code again executes:

ADC_CR2 |= (1 << 30);

inside the while(1) loop.

If it were Continuous Mode, you would have:

ADC_CR2 |= (1 << 1);    // CONT = 1

Then:

Start once
    ↓
Convert
    ↓
Convert again
    ↓
Convert again
    ↓
Convert again
    ↓
...

without needing another SWSTART.

So the code you posted is Single Conversion Mode, not Continuous Mode.

give code for continous mode to execute

In the continuous mode code I gave, we are not taking temperature data at all.

We are reading:

ADC_SQR3 = 10;

which means:

Channel 10
↓
PC0
↓
External Analog Voltage

Then:

ADC_Value = ADC_DR;

reads the ADC result.

And:

voltage = (ADC_Value * 3.3f) / 4095.0f;

converts the ADC count to voltage.

So the code is currently doing:

PC0 Voltage
    ↓
ADC
    ↓
ADC_Value
    ↓
Voltage

NOT:

Temperature Sensor
    ↓
ADC
    ↓
Temperature

If your external sensor is an LM35, then after calculating voltage:

Temperature = voltage * 100;

Because:

LM35:
10 mV/°C

25°C = 0.25V
30°C = 0.30V
50°C = 0.50V

If your sensor is not LM35, tell me the sensor name (LM35, TMP36, NTC thermistor, etc.), and I'll show the exact temperature calculation.

i am using lm35 only can i put Temperature = voltage * 100;this line after voltage line to get temprature

Yes, if you are using an LM35 connected to PC0 (ADC Channel 10), then you can do:

voltage = (ADC_Value * 3.3f) / 4095.0f;

Temperature = voltage * 100.0f;

Because LM35 characteristics are:

10 mV/°C
=
0.01 V/°C

Therefore:

Temperature=Voltage0.01Temperature=0.01Voltage​Temperature=Voltage×100Temperature=Voltage×100

### Example

If ADC gives:

Voltage = 0.25 V

Then:

Temperature = 0.25 * 100;

Result:

25°C

If:

Voltage = 0.37 V

Then:

Temperature = 0.37 * 100;

Result:

37°C

So for an LM35, place:

Temperature = voltage * 100.0f;

immediately after the voltage calculation. The previous formula:

((voltage - 0.76) / 0.0025) + 25

is only for the STM32's internal temperature sensor, not for LM35.

dont replay

👍

Top of Form

Bottom of Form