# STM32F401 UART / USART Notes

These notes are based on the STM32F401 Reference Manual USART chapter and STM32F401 Datasheet. The STM32F401 contains USART1, USART2 and USART6 peripherals for serial communication.

# 1. UART Basics

## What is UART?

UART (Universal Asynchronous Receiver Transmitter) is a hardware peripheral used for serial communication between two devices.

Examples:

- STM32 ↔ PC

- STM32 ↔ GPS Module

- STM32 ↔ Bluetooth Module

- STM32 ↔ ESP8266

UART uses only:

| Signal | Function |
| --- | --- |
| TX | Transmit Data |
| RX | Receive Data |
| GND | Common Ground |

STM32                PC/Module

TX  -------------> RX

RX <------------- TX

GND ------------- GND

## UART vs USART

### UART

Asynchronous communication only.

### USART

Universal Synchronous Asynchronous Receiver Transmitter.

Supports:

- UART mode (Asynchronous)

- USART mode (Synchronous)

STM32F401 USART supports:

- Full Duplex

- Half Duplex

- LIN

- Smart Card

- IrDA

- DMA

- Hardware Flow Control (CTS/RTS)

# 2. UART Frame Format

UART sends data bit-by-bit.

Typical frame:

Idle   Start    Data Bits       Stop
 1       0      01010101         1
         |<---- 8 bits ---->|

### 8N1 Format

Most common configuration.

- 8 Data Bits

- No Parity

- 1 Stop Bit

|Start|D0|D1|D2|D3|D4|D5|D6|D7|Stop|
   0                               1

## Example

Sending Character 'A'

ASCII:

'A' = 65 decimal
    = 0x41
    = 01000001

UART transmits LSB first:

Start

1 0 0 0 0 0 1 0

Stop

# 3. Baud Rate

## Definition

Baud rate = Number of bits transmitted per second.

Examples:

| Baud Rate | Speed |
| --- | --- |
| 9600 | 9600 bits/s |
| 19200 | 19200 bits/s |
| 115200 | 115200 bits/s |
| 1 Mbps | 1,000,000 bits/s |

## STM32 Baud Rate Generation

USART uses:

PCLK
 |
 v
Baud Generator
 |
 v
TX/RX

STM32 uses USART_BRR register for baud generation.

### Formula

For oversampling by 16:

Where:

- fCK = Peripheral Clock

- USARTDIV = Value programmed in BRR register

## Example

APB Clock:

16 MHz

Desired Baud:

9600

USARTDIV = 16000000/(16×9600)

USARTDIV = 104.16

Program BRR accordingly.

# 4. TX and RX Operation

## Transmitter (TX)

Data Flow:

CPU
 |
USART_DR
 |
TDR Buffer
 |
Shift Register
 |
TX Pin

Procedure:

- Enable USART

- Configure baud rate

- Enable TE bit

- Write data into DR register

- Hardware shifts data out

## Receiver (RX)

Data Flow:

RX Pin
 |
Shift Register
 |
RDR Buffer
 |
USART_DR
 |
CPU

Procedure:

- Enable USART

- Enable RE bit

- Wait for RXNE flag

- Read USART_DR

# 5. USART Registers

The most important USART registers are:

USART_SR
USART_DR
USART_BRR
USART_CR1
USART_CR2
USART_CR3

# 6. USART Register Diagram

## USART_SR (Status Register)

------------------------------------------------
|CTS|LBD|TXE|TC|RXNE|IDLE|ORE|NF|FE|PE|
------------------------------------------------

Important Bits:

| Bit | Meaning |
| --- | --- |
| TXE | TX buffer empty |
| TC | Transmission complete |
| RXNE | Receive data available |
| ORE | Overrun Error |
| FE | Framing Error |
| PE | Parity Error |

## USART_DR (Data Register)

------------------
| DR[8:0]        |
------------------

Used for:

- Writing TX data

- Reading RX data

## USART_BRR (Baud Rate Register)

----------------------------------
| Mantissa | Fraction            |
----------------------------------

Controls baud rate generation.

## USART_CR1

------------------------------------------------------
| UE | M | TE | RE | RXNEIE | TCIE | TXEIE | ...
------------------------------------------------------

Important bits:

| Bit | Function |
| --- | --- |
| UE | USART Enable |
| TE | Transmitter Enable |
| RE | Receiver Enable |
| TXEIE | TX Interrupt |
| RXNEIE | RX Interrupt |
| TCIE | Transmission Complete Interrupt |

## USART_CR2

--------------------------
| STOP[1:0] | ...
--------------------------

Used for:

- Stop bits

- Clock polarity

- Clock phase

## USART_CR3

-----------------------------------------
| DMAT | DMAR | RTSE | CTSE | EIE |
-----------------------------------------

Used for:

- DMA

- Flow control

- Error interrupts

# 7. UART Communication Flow

## Transmission Flow

Application
    |
Write USART_DR
    |
TXE Cleared
    |
Shift Register Loads Data
    |
Data Sent on TX
    |
TXE Set
    |
TC Set

## Reception Flow

RX Pin
   |
Start Bit Detected
   |
Receive Data Bits
   |
Store in USART_DR
   |
RXNE = 1
   |
CPU Reads Data
   |
RXNE Cleared

# 8. Polling Mode

Polling means CPU continuously checks status flags.

## UART TX Polling

void UART_SendChar(char ch)
{
    while(!(USART2->SR & (1<<7)));   // TXE

    USART2->DR = ch;
}

### Explanation

Check TXE
     |
     v
TXE=1 ?
     |
   Yes
     |
Write DR
     |
Transmit

## UART RX Polling

char UART_ReadChar(void)
{
    while(!(USART2->SR & (1<<5)));   // RXNE

    return USART2->DR;
}

### Advantages

- Easy

- Good for beginners

### Disadvantages

- CPU always busy

# 9. Interrupt Mode Basics

Instead of polling:

while(RXNE==0)

USART informs CPU automatically.

## Enable RX Interrupt

USART2->CR1 |= (1<<5);   // RXNEIE

Enable NVIC interrupt.

## Interrupt Flow

Byte Received
      |
RXNE=1
      |
Interrupt Generated
      |
USART2_IRQHandler()
      |
Read DR
      |
Process Data

## Example ISR

volatile char rx_data;

void USART2_IRQHandler(void)
{
    if(USART2->SR & (1<<5))
    {
        rx_data = USART2->DR;
    }
}

# 10. Serial Communication Example

## PC → STM32

User types:

HELLO

PC UART sends:

H
E
L
L
O

STM32 receives one byte at a time.

RXNE=1
Read DR

RXNE=1
Read DR

RXNE=1
Read DR

## STM32 → PC

UART_SendChar('O');
UART_SendChar('K');

PC Terminal displays:

OK

# 11. Bare-Metal UART TX Example (STM32F401)

void UART_SendChar(char c)
{
    while(!(USART2->SR & USART_SR_TXE));

    USART2->DR = c;
}

## Send String

void UART_SendString(char *str)
{
    while(*str)
    {
        UART_SendChar(*str++);
    }
}

Usage:

UART_SendString("STM32 UART\r\n");

# 12. Bare-Metal UART RX Example

char UART_ReadChar(void)
{
    while(!(USART2->SR & USART_SR_RXNE));

    return USART2->DR;
}

Usage:

char c;

c = UART_ReadChar();

# Quick Interview Questions

### What is UART?

A serial asynchronous communication protocol using TX and RX lines.

### Difference between UART and USART?

UART = Asynchronous only.

USART = Synchronous + Asynchronous.

### Which register sets baud rate?

USART_BRR

### Which flag indicates transmit buffer empty?

TXE

### Which flag indicates received data available?

RXNE

### Which register contains transmitted/received data?

USART_DR

### Which bits enable TX and RX?

TE -> Transmitter Enable
RE -> Receiver Enable

### Which bit enables USART?

UE

These are the most important STM32F401 USART concepts needed before moving to complete bare-metal initialization and full USART2 driver development.

### How does STM32 USART support Full Duplex?

Full duplex means:

STM32 TX  ------> Device RX

STM32 RX  <------ Device TX

Transmission and reception happen simultaneously.

STM32 USART has:

- Separate Transmit Shift Register

- Separate Receive Shift Register

- Separate TX pin

- Separate RX pin

So hardware can send and receive at the same time.

# Which Registers Enable Full Duplex?

There is no dedicated FULL_DUPLEX bit.

Full duplex is achieved by enabling both:

### TE (Transmitter Enable)

in USART_CR1

USART2->CR1 |= (1<<3);   // TE = 1

### RE (Receiver Enable)

in USART_CR1

USART2->CR1 |= (1<<2);   // RE = 1

### Both Together

USART2->CR1 |= (1<<3);   // TE
USART2->CR1 |= (1<<2);   // RE

or

USART2->CR1 |= (1<<3) | (1<<2);

Register view:

USART_CR1

-------------------------------------------------
| ... | TE | RE | ... |
-------------------------------------------------
        3    2

# Example

Suppose PC sends:

HELLO

At the same time STM32 sends:

OK

Hardware operation:

TX Shift Register -----> TX Pin -----> PC

RX Pin -----> RX Shift Register -----> DR

Both paths are independent.

USART

      +------------------+

TX -->| TX Shift Register|---->

      |                  |

<-----| RX Shift Register|<---- RX

      +------------------+

Therefore TX does not block RX and RX does not block TX.

# What if only TE=1?

USART2->CR1 |= (1<<3);   // TE=1

Result:

Transmit ✔
Receive ✘

# What if only RE=1?

USART2->CR1 |= (1<<2);   // RE=1

Result:

Transmit ✘
Receive ✔

# What if TE=1 and RE=1?

USART2->CR1 |= (1<<3) | (1<<2);

Result:

Transmit ✔
Receive ✔

= Full Duplex

This is the normal UART configuration used in most STM32 applications. STM32 USART provides separate enable bits for transmitter and receiver, which is one of its listed features.

So for interviews:

Q: Which register bit enables full duplex mode in STM32 USART?

Answer: There is no separate full-duplex bit. Full duplex is obtained by setting both TE (bit 3) and RE (bit 2) in USART_CR1 while USART is enabled (UE=1).

Yes, exactly.

UART/USART hardware contains two separate paths:

USART

          TRANSMITTER
      +------------------+
CPU --> USART_DR (TDR) --> TX Shift Register --> TX Pin


          RECEIVER
      +------------------+
RX Pin --> RX Shift Register --> USART_DR (RDR) --> CPU

Because TX and RX have separate shift registers, transmission and reception can occur simultaneously (full duplex).

### SPI Comparison

SPI is also full duplex, but its architecture is slightly different.

Master SPI

MOSI -----------------> Slave

MISO <----------------- Slave

SPI also has:

- Transmit Shift Register

- Receive Shift Register

so during every clock pulse:

Master sends 1 bit
Master receives 1 bit

simultaneously.

### Key Difference

#### USART

TX and RX are largely independent.

TX can run alone
RX can run alone
TX + RX can run together

You can set:

TE = 1, RE = 0   // TX only
TE = 0, RE = 1   // RX only
TE = 1, RE = 1   // Full duplex

#### SPI

In SPI, transmission and reception are tied to the same clock.

No clock --> No transfer

When master transmits:

MOSI shifts out
MISO shifts in

at the same clock edges.

So SPI is inherently a shift-register exchange mechanism:

Master TX Shift Register  <---->  Slave RX Shift Register

Master RX Shift Register  <---->  Slave TX Shift Register

One bit goes out while one bit comes in.

### Interview Answer

Q: Does UART have separate shift registers like SPI?

Answer:
Yes. STM32 USART contains a separate transmit shift register and a separate receive shift register. This allows independent transmission and reception and enables full-duplex communication. SPI also has separate transmit and receive shift registers, but SPI transmission and reception are synchronized by the shared clock, whereas UART TX and RX operate independently using start/stop bit timing.

If you're asking "How is UART half-duplex achieved if it has separate TX and RX shift registers?", that's a very important STM32 USART concept.

### Normal Full-Duplex UART

STM32 TX ---------> Device RX

STM32 RX <--------- Device TX

Uses two wires:

- TX pin

- RX pin

Both TE and RE can be enabled.

# Half-Duplex UART in STM32

In half-duplex mode, STM32 uses only one communication wire.

Single Wire
STM32  <----------------> Device

The same line is used for:

- Transmitting

- Receiving

But not at the same time.

### How does STM32 do this?

STM32 USART has a special bit:

HDSEL (Half Duplex Select) in USART_CR3.

USART2->CR3 |= (1<<3);   // HDSEL = 1

When HDSEL = 1:

RX pin is no longer used

TX pin becomes:

Bidirectional Data Line

Conceptually:

Normal Mode

TX ------>
RX <------

Half Duplex

TX <------>

# Internal Hardware View

### Full Duplex

TX Shift Register --> TX Pin

RX Pin --> RX Shift Register

### Half Duplex

Single Pin

TX Shift Register --->|
                      |---- TX Pin
RX Shift Register <---|

Both shift registers still exist internally.

The difference is:

- One shared pin

- Only one direction active at a time

# Example

Suppose STM32 wants to send:

HELLO

Enable transmitter:

TE = 1
RE = 0

Transmit data.

After transmission:

TE = 0
RE = 1

Now listen for reply:

OK

# Why can't TX and RX happen simultaneously?

Because there is only one wire.

Imagine:

STM32 ---- Wire ---- Device

If both sides drive the wire simultaneously:

STM32 -> HIGH
Device -> LOW

Bus contention occurs.

Therefore:

Transmit Phase
      OR
Receive Phase

Only one at a time.

# Real-World Examples

Half-duplex communication is used in:

- RS-485 networks

- LIN Bus

- Single-wire UART

- Some sensor networks

### Interview Answer

Q: How is half-duplex achieved in STM32 USART?

Answer:
STM32 USART supports half-duplex mode using the HDSEL bit in USART_CR3. When HDSEL is set, the RX pin is disconnected and the TX pin becomes a bidirectional data line. The USART still contains separate transmit and receive shift registers, but transmission and reception share the same physical pin, so only one direction can be active at a time.

This is very similar to how RS-485 half-duplex communication works, which is a common interview follow-up question.

You are asking about LIN (Local Interconnect Network) support in STM32 USART.

LIN is actually built on top of UART.

# What is LIN?

LIN = Local Interconnect Network

It is a low-cost serial communication protocol mainly used in automobiles.

Examples:

- Power windows

- Door locks

- Seat control

- Mirror adjustment

- Rain sensors

- Sunroof

Instead of using expensive CAN for everything, car manufacturers use LIN for slower devices.

# LIN Network

Master
              |
              |
      -------------------
      |    |    |    |
    Slave Slave Slave Slave

Example:

Body Control Module
        |
   -------------
   |     |     |
Door  Mirror  Window

Only one master exists.

Multiple slaves exist.

# Physical Layer

LIN uses:

1 Data Wire
1 Ground

Unlike UART:

TX
RX
GND

LIN uses a single shared bus.

# Why STM32 USART Supports LIN?

Because LIN frame format is almost the same as UART.

Normal UART frame:

Start + Data + Stop

LIN frame:

Break + Sync + ID + Data + Checksum

STM32 USART hardware can automatically detect the LIN break field.

# LIN Frame Structure

| Break | Sync | ID | Data | Checksum |

Example:

| 13 bits 0 | 0x55 | 0x12 | 01 02 03 | CS |

# Break Field

This is the most important part.

Normal UART start bit:

0

LIN Break:

0000000000000

(13 consecutive zeros)

This tells all slave nodes:

"Attention!
A new LIN frame is starting."

# STM32 LIN Detection

USART hardware has:

### LINEN bit

Located in:

USART_CR2

Enable LIN mode:

USART2->CR2 |= (1<<14); // LINEN

### LBD Flag

Located in:

USART_SR

LBD means:

LIN Break Detection

When break detected:

LBD = 1

### LBDIE

Located in:

USART_CR2

Enable interrupt on break detection.

USART2->CR2 |= (1<<6); // LBDIE

# How Break Detection Works

STM32 watches RX line.

If it sees:

0000000000

or

00000000000

continuous dominant bits,

then:

LBD = 1

Break detected.

# LIN Transmission Example

Master sends:

Break
Sync
ID

Example:

0000000000000
0x55
0x22

Slave sees:

LBD = 1

Then receives:

Sync
ID

and responds if ID matches.

# Important LIN Registers

## USART_CR2

-----------------------------------------
| LINEN | STOP | ... | LBDIE | LBDL |
-----------------------------------------

### LINEN

Enable LIN mode.

### LBDIE

Break interrupt enable.

### LBDL

Break detection length:

0 = 10 bits
1 = 11 bits

## USART_SR

----------------------------------
| LBD | TXE | TC | RXNE | ...
----------------------------------

### LBD

Set when LIN break detected.

# LIN vs UART

| Feature | UART | LIN |
| --- | --- | --- |
| Cost | Low | Very Low |
| Wires | TX,RX | Single Bus |
| Master-Slave | No | Yes |
| Break Field | No | Yes |
| Addressing | No | Yes |
| Automotive | Rare | Very Common |

This is where many beginners get confused.

## STM32 Peripheral Name = USART

STM32F401 has:

- USART1

- USART2

- USART6

There is no separate UART peripheral in STM32F401.

## Then how does it work as UART?

A USART can operate in two modes:

### 1. Asynchronous Mode (UART Mode)

TX
RX
GND

No clock line.

This is what we commonly call UART.

Example:

USART2->CR1 |= (1<<13);   // UE
USART2->CR1 |= (1<<3);    // TE
USART2->CR1 |= (1<<2);    // RE

No clock configuration is done.

Now USART behaves like a UART.

USART Hardware
      |
      v
Asynchronous Mode
      |
      v
UART Communication

### 2. Synchronous Mode (USART Mode)

Here an extra clock pin is used.

TX
RX
CK (Clock)
GND

Frame:

Clock ----->

Data  ------>

Receiver samples using clock

To enable synchronous mode:

Enable clock output bit CLKEN in USART_CR2.

USART2->CR2 |= (1<<11);   // CLKEN

Now USART uses:

TX
RX
CK

and behaves as a true USART.

## Hardware View

USART Peripheral
             +------------------+
             |                  |
             | Async Logic      |
             | Sync Logic       |
             +------------------+
                     |
       --------------------------------
       |                              |
UART Mode                    USART Mode
(Async)                      (Sync)

## How to Know Which Mode We Are Using?

### If CLKEN = 0

USART2->CR2 &= ~(1<<11);

Mode:

UART (Asynchronous)

Only:

TX
RX

are used.

### If CLKEN = 1

USART2->CR2 |= (1<<11);

Mode:

USART (Synchronous)

Pins used:

TX
RX
CK

## In Your UART Programs

When you write:

USART2->BRR = 0x0683;
USART2->CR1 |= (1<<13);
USART2->CR1 |= (1<<3);
USART2->CR1 |= (1<<2);

and never set CLKEN,

you are actually using:

USART2 Peripheral
        ↓
UART Mode

So we usually say:

"UART communication using USART2."

## Interview Answer

Q: STM32F401 has USART peripherals. How do we use UART?

Answer:
The STM32F401 contains USART peripherals. When the synchronous clock output (CLKEN bit in USART_CR2) is disabled, the USART operates in asynchronous mode and behaves as a UART. When CLKEN is enabled, the peripheral operates in synchronous USART mode using the CK clock pin in addition to TX and RX.

what is the spped of this onre

If by "this one" you mean the STM32 USART operating in synchronous mode (USART mode), then the speed depends on the peripheral clock and baud rate configuration.

### Asynchronous Mode (UART)

Common baud rates:

9600
19200
38400
57600
115200
460800
921600

STM32F401 can operate much higher than 115200 if clock accuracy allows.

### Synchronous Mode (USART)

In synchronous mode, the transmitter also outputs a clock on the CK pin.

TX ---- Data
RX ---- Data
CK ---- Clock

The maximum clock frequency is limited by the USART peripheral clock.

For STM32F401:

- USART1 and USART6 are on APB2

- USART2 is on APB1

The USART can generate synchronous clock rates of several MHz depending on the configured peripheral clock and divider settings.

Conceptually:

USART Clock
      ↓
 Baud Rate Generator
      ↓
CK Output Frequency

The data rate equals the configured baud rate.

### Comparison

| Interface | Typical Speed |
| --- | --- |
| UART | 9.6 kbps – 1 Mbps+ |
| USART Sync | Several Mbps |
| SPI | Tens of Mbps |
| I2C | 100 kbps – 3.4 Mbps |

SPI is usually much faster because it was designed specifically for synchronous high-speed data transfer.

### For STM32F401 Interviews

Q: Which is faster, UART or USART synchronous mode?

Answer:
USART synchronous mode is generally faster and more reliable because a clock signal (CK) is transmitted along with the data, eliminating the need for the receiver to recover timing from start/stop bits. UART mode is asynchronous and its practical speed is limited by clock accuracy and baud-rate matching.

If you're asking for the exact maximum baud rate achievable on STM32F401 USART1/USART2/USART6, I can derive it from the APB1/APB2 clock frequencies and the baud-rate generator formula.

This is one of the most important UART concepts. Let's go step by step and understand how STM32 generates the baud rate using the BRR register.

# 1. Why do we need a Baud Generator?

Suppose you want UART communication at:

9600 baud

This means:

9600 bits per second

So every bit must occupy:

1/9600

= 104.16 µs

The USART hardware must generate this timing accurately.

It gets timing from the peripheral clock (PCLK).

APB Clock (PCLK)
       |
       v
Baud Rate Generator
       |
       v
TX/RX Logic

# 2. Where does USART get its clock?

For STM32F401:

### USART1, USART6

APB2 Clock

### USART2

APB1 Clock

Example:

PCLK1 = 16 MHz

This means:

16,000,000 clock pulses/sec

# 3. Problem

UART needs:

9600 bits/sec

But clock is:

16,000,000 pulses/sec

We must divide the clock.

16 MHz
   |
   v
Divider
   |
   v
9600 baud

The divider value is called:

USARTDIV

# 4. Baud Rate Formula

For Oversampling by 16:

Where:

| Parameter | Meaning |
| --- | --- |
| fCK | Peripheral Clock |
| 16 | Oversampling |
| USARTDIV | Divider value |

# 5. What is Oversampling?

UART receiver must determine:

Start Bit
Data Bits
Stop Bit

To improve accuracy, STM32 samples each bit multiple times.

### Oversampling by 16

One UART Bit

|-------------------|

16 internal samples

x x x x x x x x x x x x x x x x

The receiver decides the bit value using the middle samples.

This makes UART robust against noise.

# 6. Calculate USARTDIV

Suppose:

PCLK = 16 MHz

Baud = 9600

Formula:

USARTDIV = 16000000/(16×9600)

Calculate:

USARTDIV = 104.1667

# 7. BRR Register Structure

USARTDIV is stored inside:

USART_BRR

Structure:

--------------------------------
| Mantissa | Fraction          |
--------------------------------
15:4         3:0

For oversampling by 16:

USARTDIV = Mantissa + Fraction/16

# 8. Split the Value

We got:

104.1667

Split:

### Mantissa

104

### Fraction

0.1667

Now:

Fraction × 16

= 0.1667 × 16

= 2.667

Round:

3

So:

Mantissa = 104
Fraction = 3

# 9. Program BRR

BRR format:

Mantissa = 104 = 0x68

Fraction = 3

Store:

BRR = (104 << 4) | 3

USART2->BRR = (104 << 4) | 3;

Result:

0x0683

This is the famous value you often see in examples.

# 10. Hardware Internally

Suppose:

BRR = 0x0683

Hardware interprets:

Mantissa = 104

Fraction = 3

Therefore:

USARTDIV

= 104 + 3/16

= 104.1875

Now baud generator produces approximately:

9600 baud

# 11. Why Not Store 9600 Directly?

Because UART hardware doesn't need the baud rate.

It needs the divider.

Hardware thinks:

Clock = 16 MHz

Divider = 104.1875

Generate output timing

So BRR stores the divider, not the baud rate.

# 12. Complete Flow

PCLK = 16 MHz
      |
      v
Calculate USARTDIV

USARTDIV = 104.1667
      |
      v
Mantissa = 104
Fraction = 3
      |
      v
BRR = 0x0683
      |
      v
Baud Generator
      |
      v
9600 Baud Timing
      |
      v
TX/RX Logic

# Interview Question

### Why is BRR divided into Mantissa and Fraction?

Because the required baud-rate divider is often not an integer. The mantissa stores the integer part of USARTDIV, while the fraction stores the fractional part, allowing STM32 to generate baud rates more accurately.

### For 16 MHz clock and 9600 baud, what is BRR?

USARTDIV = 104.1667

Mantissa = 104

Fraction = 3

BRR = 0x0683

This is one of the most commonly asked STM32 UART interview calculations.

Excellent doubt. This shows you're thinking about the hardware design.

You're asking:

If USART1, USART2, and USART6 can all generate the same baud rates (9600, 115200, 1 Mbps, etc.), why do they use different clocks (APB1 and APB2)?

The answer is:

The baud rate is the final output speed. The peripheral clock is the input source used to generate that speed.

Think of it like this:

Water Tank (Clock)
      |
      v
Valve (BRR Divider)
      |
      v
Water Flow (Baud Rate)

Different tank sizes can still produce the same flow rate if you adjust the valve.

## Example 1: USART2

Suppose:

PCLK1 = 16 MHz
Baud  = 9600

USARTDIV:

16000000 / (16 × 9600)

= 104.16

BRR stores about 104.16.

## Example 2: USART1

Suppose:

PCLK2 = 84 MHz
Baud  = 9600

USARTDIV:

84000000 / (16 × 9600)

= 546.875

BRR stores about 546.875.

Notice:

USART1 Clock = 84 MHz
USART2 Clock = 16 MHz

but both finally produce:

9600 baud

because the divider value is different.

The division value (USARTDIV) is calculated directly from the baud rate formula.

For STM32 USART with oversampling by 16:

Where:

- fCK = USART peripheral clock

- Baud Rate = desired UART speed

# Example 1: 16 MHz Clock, 9600 Baud

Given:

fCK = 16,000,000

Baud = 9600

Calculate:

USARTDIV

= 16000000 / (16 × 9600)

= 16000000 / 153600

= 104.1667

Split into:

Mantissa = 104

Fraction = 0.1667 × 16
         = 2.667
         ≈ 3

Therefore:

BRR = (104 << 4) | 3

    = 0x0683

# Example 2: 84 MHz Clock, 115200 Baud

Given:

fCK = 84,000,000

Baud = 115200

Calculate:

USARTDIV

= 84000000 / (16 × 115200)

= 45.5729

Split:

Mantissa = 45

Fraction = 0.5729 × 16

         = 9.16

         ≈ 9

BRR:

BRR = (45 << 4) | 9

    = 0x2D9

# Why Mantissa and Fraction?

Hardware cannot store:

104.1667

directly.

So STM32 stores:

104 + 3/16

= 104.1875

which is very close.

# Interview Shortcut

For oversampling by 16:

1. USARTDIV = Clock / (16 × Baud)

2. Mantissa = Integer Part

3. Fraction = Decimal Part × 16

4. BRR = (Mantissa << 4) | Fraction

### Quick Question

If:

PCLK = 42 MHz
Baud = 115200

try calculating USARTDIV, Mantissa, Fraction, and BRR yourself, and I'll verify it step by step.

Ah! Oversampling by 16 is one of the most important UART receiver concepts.

Let's understand why STM32 samples a bit 16 times instead of just once.

# Problem

Suppose the sender transmits:

1 bit = 104.16 µs   (9600 baud)

If the receiver samples only once:

Sample
           ↓
|-------------------|
       Bit = 1

What if:

- Noise occurs exactly at the sample instant?

- Sender and receiver clocks differ slightly?

Then the bit may be received incorrectly.

# Solution: Oversampling

Instead of sampling once, STM32 samples multiple times during the bit period.

For Oversampling by 16:

One UART Bit

|--------------------------------|

1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16
x x x x x x x x x  x  x  x  x  x  x  x

The USART uses the middle samples to decide whether the bit is 0 or 1.

# Example

Suppose transmitted bit is:

1

Samples received:

1 1 1 1 1 1 1 1 1 1 1 0 1 1 1 1

One sample got corrupted by noise.

But most samples are:

1

So USART decides:

Received Bit = 1

Correctly.

# How does STM32 generate these 16 samples?

Suppose:

Baud Rate = 9600

Bit time:

1 / 9600

= 104.16 µs

Oversampling by 16 means:

104.16 µs / 16

= 6.51 µs

USART internally samples every:

6.51 µs

giving 16 samples per bit.

# Why does the formula contain 16?

Remember:

The 16 comes from the oversampling factor.

Hardware needs a clock that is:

16 × Baud Rate

because it wants 16 samples for every bit.

Example:

Baud = 9600

Sampling clock:

9600 × 16

= 153600 Hz

So the USART first generates:

153600 Hz

and then uses that to create the actual 9600-bit timing.

# Oversampling by 8

STM32 also supports:

OVER8 = 1

in USART_CR1.

Then:

1 Bit
|--------------------|

1 2 3 4 5 6 7 8
x x x x x x x x

Only 8 samples per bit.

Formula becomes:

# Comparison

| Oversampling | Accuracy | Max Baud Rate |
| --- | --- | --- |
| 16 | Better | Lower |
| 8 | Less Accurate | Higher |

# Internal UART Receiver

RX Pin
   |
   v
Start Bit Detector
   |
   v
Oversampling Clock
(16 samples/bit)
   |
   v
Majority Vote Logic
   |
   v
Shift Register

### Interview Question

Q: Why does STM32 USART use oversampling by 16?

Answer:
Oversampling by 16 means the receiver samples each UART bit 16 times. This improves noise immunity, clock tolerance, and bit detection accuracy. The USART uses these samples to determine the correct bit value before shifting it into the receive register.

This is exactly why the baud-rate formula contains the factor 16.

Perfect. Let's take a real STM32F401 scenario.

Suppose you want to use USART1 at 9600 baud.

# Step 1: Where does USART1 get its clock?

From the STM32 clock tree:

HSI = 16 MHz
      |
      v
     PLL
      |
      v
SYSCLK = 84 MHz
      |
      v
APB2 Prescaler = 1
      |
      v
PCLK2 = 84 MHz
      |
      v
USART1

USART1 is connected to APB2 bus.

Therefore:

USART1 Clock (fCK)

= PCLK2

= 84 MHz

# Step 2: How does USART know this clock?

It doesn't "know".

You configure RCC registers.

Example:

RCC->APB2ENR |= (1<<4);   // Enable USART1 clock

This only enables the peripheral clock.

Now USART1 receives:

84 MHz

internally.

# Step 3: Desired Baud Rate

Suppose:

Baud = 9600

# Step 4: Calculate USARTDIV

Formula:

USARTDIV

= fCK/(16 × Baud)

Substitute values:

= 84000000/(16 × 9600)

= 84000000/153600

= 546.875

# Step 5: Split into Mantissa and Fraction

546.875

Mantissa = 546

Fraction = 0.875

Fraction field:

0.875 × 16

= 14

So:

Mantissa = 546

Fraction = 14

# Step 6: Calculate BRR

BRR = (546 << 4) | 14;

Let's calculate:

546 = 0x222

BRR

= 0x222E

Program:

USART1->BRR = 0x222E;

# Step 7: What Happens Internally?

USART receives:

84 MHz clock

Hardware sees:

BRR = 546.875

Internally:

84 MHz
   |
   v
Divide by 546.875
   |
   v
153600 Hz

Remember:

153600

= 9600 × 16

because of oversampling by 16.

Then:

153600 Hz
    |
    v
16 samples per bit
    |
    v
9600 Baud

# Complete Picture

HSI
 |
 v
PLL
 |
 v
84 MHz SYSCLK
 |
 v
APB2
 |
 v
PCLK2 = 84 MHz
 |
 v
USART1
 |
 v
BRR = 546.875
 |
 v
Sampling Clock = 153600 Hz
 |
 v
9600 Baud
 |
 v
TX/RX Pins

### Important Interview Point

When someone asks:

"How does USART1 get its clock?"

You should answer:

USART1 gets its input clock from PCLK2 (APB2 peripheral clock). The RCC clock tree determines PCLK2. The baud-rate generator then divides PCLK2 according to the BRR register value to produce the required UART baud rate.

This clock-tree → PCLK2 → USART1 → BRR → Baud flow is exactly how every STM32 USART works.

Excellent question. This is exactly where many engineers get confused between clock frequency and baud rate.

# First Understand the Difference

### Clock (CK)

The clock is used to tell the receiver:

"When should I sample the data?"

Example:

CK:  _|‾|_|‾|_|‾|_|‾|_

DATA: 1   0   1   1

Each clock edge tells the receiver to read one bit.

### Baud Rate

Baud rate tells:

"How fast are bits transferred?"

Example:

9600 baud

= 9600 bits/sec

# In UART (Asynchronous)

There is NO clock wire.

TX
RX

Receiver doesn't get a clock from transmitter.

So both sides agree beforehand:

9600 baud

or

115200 baud

Then both generate timing internally.

STM32                     PC

9600 baud  <------->  9600 baud

No clock pin exists.

# In USART Synchronous Mode

Now we have:

TX
RX
CK

The transmitter sends a clock.

STM32
 |
 +---- TX
 |
 +---- CK --------> Receiver

# Then Why Do We Still Configure Baud Rate?

Because the clock itself must have a frequency.

Suppose you configure:

USART1->BRR = ...

for

9600 baud

The USART generates:

CK = 9600 Hz

approximately.

Then:

Clock Edge 1 -> Send Bit 1

Clock Edge 2 -> Send Bit 2

Clock Edge 3 -> Send Bit 3

# Think of Baud Rate as Clock Frequency

In synchronous mode:

Baud Rate
     ↓
Determines
     ↓
CK Frequency

For example:

Baud = 1 Mbps

Then:

CK ≈ 1 MHz

because one clock pulse transfers one bit.

# Why Not Just Specify CK Directly?

STM32 USART hardware is designed around the baud generator.

Internally:

PCLK
  |
  v
Baud Generator (BRR)
  |
  +---- TX Timing
  |
  +---- CK Output

So you always configure:

BRR

and hardware derives:

Data Timing
Clock Timing

from it.

# Real Example

Suppose:

USART1 Clock = 84 MHz

Desired Speed = 1 Mbps

You calculate BRR.

Hardware generates:

TX bit rate = 1 Mbps

CK frequency ≈ 1 MHz

Receiver uses CK directly.

Sure. Let's understand USART_CR1 in detail because this is the most important USART control register.

# USART_CR1 (Control Register 1)

This register controls:

- USART Enable
- Transmitter
- Receiver
- Interrupts
- Word Length
- Parity
- Oversampling

Simplified view:

31                              0
---------------------------------------------------------
| UE | M | WAKE | PCE | PS | PEIE | TXEIE | TCIE |
---------------------------------------------------------
| RXNEIE | TE | RE | RWU | SBK |
---------------------------------------------------------

For beginners, focus on these bits:

--------------------------------------------------
| UE | M | TE | RE | RXNEIE | TCIE | TXEIE |
--------------------------------------------------

# 1. UE (USART Enable)

### Bit Position

UE = Bit 13

### Purpose

Turns USART ON or OFF.

USART2->CR1 |= (1<<13);

Meaning:

UE = 1

USART Enabled

Without UE:

TE = 1
RE = 1

Still USART won't work.

Think of UE as the main power switch.

UE=0

USART OFF

UE=1

USART ON

# 2. TE (Transmitter Enable)

### Bit Position

TE = Bit 3

### Purpose

Enables transmission.

USART2->CR1 |= (1<<3);

Data flow:

CPU
 |
 DR
 |
 TX Shift Register
 |
 TX Pin

If:

TE=0

Then:

Cannot transmit

Even if you write:

USART2->DR='A';

nothing appears on TX pin.

# 3. RE (Receiver Enable)

### Bit Position

RE = Bit 2

### Purpose

Enables reception.

USART2->CR1 |= (1<<2);

Data flow:

RX Pin
 |
 RX Shift Register
 |
 DR
 |
 CPU

If:

RE=0

Incoming bytes are ignored.

# TE and RE Together

Most applications use:

USART2->CR1 |= (1<<3)|(1<<2);

Result:

TE=1
RE=1

Full Duplex UART

# 4. RXNEIE (RX Interrupt Enable)

### Bit Position

RXNEIE = Bit 5

RXNE means:

Receive Data Register Not Empty

When a byte arrives:

RXNE = 1

Normally polling:

while(!(USART2->SR & (1<<5)));

Interrupt method:

USART2->CR1 |= (1<<5);

Now hardware automatically generates:

USART2 Interrupt

## Flow

Byte Received
      |
RX Shift Register
      |
DR Loaded
      |
RXNE=1
      |
Interrupt Generated
      |
USART2_IRQHandler()

# 5. TCIE (Transmission Complete Interrupt Enable)

### Bit Position

TCIE = Bit 6

TC means:

Transmission Complete

When last bit leaves TX pin:

TC=1

If:

USART2->CR1 |= (1<<6);

then:

Transmission Finished
       |
Interrupt Generated

## Flow

Write DR
    |
Shift Register
    |
Last Bit Sent
    |
TC=1
    |
Interrupt

Useful for:

- RS485

- DMA transfers

- Packet completion

# 6. TXEIE (TX Empty Interrupt Enable)

### Bit Position

TXEIE = Bit 7

TXE means:

Transmit Data Register Empty

After hardware moves data:

DR
 |
Shift Register

DR becomes empty.

TXE = 1

If TXEIE enabled:

USART2->CR1 |= (1<<7);

Interrupt occurs.

## Flow

DR = 'A'
      |
Hardware Moves Data
      |
DR Empty
      |
TXE=1
      |
Interrupt

Used when sending long strings.

# 7. M (Word Length)

### Bit Position

M = Bit 12

Selects number of data bits.

### M=0

8-bit data

Frame:

Start D0 D1 D2 D3 D4 D5 D6 D7 Stop

Most common.

### M=1

9-bit data

Frame:

Start D0 D1 D2 D3 D4 D5 D6 D7 D8 Stop

Used in:

- Multi-processor communication

- Address/data protocols

- Not exactly. This is a very common confusion.

- CPHA does NOT directly mean:

- CPHA = 0 -> Rising Edge
CPHA = 1 -> Falling Edge

- That is wrong.

- The actual meaning is:

- CPHA = 0 -> Sample on FIRST edge

CPHA = 1 -> Sample on SECOND edge

- Now whether the first edge is rising or falling depends on CPOL.

## Case 1: CPOL = 0

- Clock idle state is LOW.

- Clock

_____|‾‾‾|_____|‾‾‾|_____

      ↑     ↑
   Rising Falling

- Here:

### CPHA = 0

- Sample on first edge:

- _____|‾‾‾|_____

      ↑
    Sample

- So:

- CPOL=0
CPHA=0

Sample on Rising Edge

### CPHA = 1

- Sample on second edge:

- _____|‾‾‾|_____

            ↓
         Sample

- So:

- CPOL=0
CPHA=1

Sample on Falling Edge

## Case 2: CPOL = 1

- Clock idle state is HIGH.

- Clock

‾‾‾‾|____|‾‾‾‾|____

      ↓      ↑
   Falling Rising

- Here:

### CPHA = 0

- Sample on first edge:

- ‾‾‾‾|____

      ↓
    Sample

- So:

- CPOL=1
CPHA=0

Sample on Falling Edge

### CPHA = 1

- Sample on second edge:

- ‾‾‾‾|____|‾‾‾‾

            ↑
          Sample

- So:

- CPOL=1
CPHA=1

Sample on Rising Edge

## Complete Table

| CPOL | CPHA | Sample Edge |
| --- | --- | --- |
| 0 | 0 | Rising |
| 0 | 1 | Falling |
| 1 | 0 | Falling |
| 1 | 1 | Rising |

### Easy Interview Answer

- CPHA selects the first or second clock edge for sampling.

- CPOL determines whether that first edge is rising or falling.

- Therefore, you must look at both CPOL and CPHA together to know the actual sampling edge.

- USART\_CR3 ---------- -----------------------------------------| DMAT | DMAR | RTSE | CTSE | EIE |----------------------------------------- Used for: * DMA * Flow control * Error interrupts

- explain this one

- Excellent. USART_CR3 (Control Register 3) contains advanced USART features.

- For STM32F401, the most important bits are:

- --------------------------------------------------
| DMAT | DMAR | RTSE | CTSE | EIE |
--------------------------------------------------

- Let's understand each with real scenarios.

# 1. DMAT (DMA Enable for Transmission)

### Problem Without DMA

- Suppose you want to send:

- "HELLO WORLD"

- CPU sends each byte:

- while(!(USART2->SR & TXE));
USART2->DR='H';

while(!(USART2->SR & TXE));
USART2->DR='E';

while(!(USART2->SR & TXE));
USART2->DR='L';
...

- CPU is busy all the time.