#include <stdint.h>

/* Base Addresses */

#define RCC_BASE        0x40023800UL

#define GPIOA_BASE      0x40020000UL

#define USART1_BASE     0x40011000UL

/* RCC Registers */

#define RCC_AHB1ENR     (*(volatile uint32_t *)(RCC_BASE + 0x30))

#define RCC_APB2ENR     (*(volatile uint32_t *)(RCC_BASE + 0x44))

/* GPIOA Registers */

#define GPIOA_MODER     (*(volatile uint32_t *)(GPIOA_BASE + 0x00))

#define GPIOA_AFRH      (*(volatile uint32_t *)(GPIOA_BASE + 0x24))

/* USART1 Registers */

#define USART1_SR       (*(volatile uint32_t *)(USART1_BASE + 0x00))

#define USART1_DR       (*(volatile uint32_t *)(USART1_BASE + 0x04))

#define USART1_BRR      (*(volatile uint32_t *)(USART1_BASE + 0x08))

#define USART1_CR1      (*(volatile uint32_t *)(USART1_BASE + 0x0C))

void GPIOA_Init(void)

{

volatile uint32_t delay;

/* Enable GPIOA Clock */

RCC_AHB1ENR |= (1 << 0);

/* Small delay for clock stabilization */

delay = RCC_AHB1ENR;

(void)delay;

/* Configure PA9 (TX) and PA10 (RX) to Alternate Function Mode */

GPIOA_MODER &= ~((3 << 18) | (3 << 20));

GPIOA_MODER |=  ((2 << 18) | (2 << 20));

/* Select AF7 (USART1) for PA9 and PA10 */

GPIOA_AFRH &= ~((0xF << 4) | (0xF << 8));

GPIOA_AFRH |=  ((7 << 4)   | (7 << 8));

}

void USART1_Init(void)

{

volatile uint32_t delay;

/* Enable USART1 Clock */

RCC_APB2ENR |= (1 << 4);

/* Small delay for clock stabilization */

delay = RCC_APB2ENR;

(void)delay;

/* Baud Rate = 9600, assuming USART clock = 16 MHz */

USART1_BRR = 0x0683;

/* UE=1, TE=1, RE=1 */

USART1_CR1 = (1 << 13) | (1 << 3) | (1 << 2);

}

void USART1_TxChar(char ch)

{

while (!(USART1_SR & (1 << 7)));   /* Wait until TXE = 1 */

USART1_DR = ch;

}

char USART1_RxChar(void)

{

while (!(USART1_SR & (1 << 5)));   /* Wait until RXNE = 1 */

return (char)(USART1_DR & 0xFF);

}

void USART1_TxString(char *str)

{

while (*str)

{

USART1_TxChar(*str++);

}

}

char data_byte;

int main(void)

{

GPIOA_Init();

USART1_Init();

USART1_TxString("USART1 Ready\r\n");

while (1)

{

data_byte = USART1_RxChar();

USART1_TxChar(data_byte);

}

}









Yes.

In your code:

void USART1_TxChar(char ch)
{
    while (!(USART1_SR & (1 << 7)));   // Wait until TXE=1
    USART1_DR = ch;                    // Send one character
}

when you execute:

USART1_DR = ch;

you are writing one character into the USART transmit data register.

For example:

USART1_TxChar('H');

puts 'H' into USART1_DR.

The USART hardware then:

USART1_DR
    ↓
Shift Register
    ↓
TX Pin (PA9)
    ↓
USB-UART Converter
    ↓
PC COM Port
    ↓
Tera Term

### Example: Sending a String

When you do:

USART1_TxString("HELLO");

the function runs:

USART1_TxChar('H');
USART1_TxChar('E');
USART1_TxChar('L');
USART1_TxChar('L');
USART1_TxChar('O');

One character at a time.

The USART hardware converts each character into UART frames and sends them serially.

### What Tera Term Receives

For character 'H':

ASCII value:

'H' = 0x48

UART frame (8N1):

Idle = 1

Start bit = 0
Data bits  = 0 0 0 1 0 0 1 0   (LSB first)
Stop bit   = 1

This frame travels on the TX wire.

Tera Term receives the frame, reconstructs 0x48, and displays:

H

### For Your Program

After reset:

USART1_TxString("USART1 Ready\r\n");

Tera Term displays:

USART1 Ready

Then:

while(1)
{
    data_byte = USART1_RxChar();
    USART1_TxChar(data_byte);
}

Suppose you type:

A

in Tera Term.

Flow:

Tera Term
    ↓
PC COM Port
    ↓
RX Pin (PA10)
    ↓
USART1_RxChar()

data_byte becomes:

'A'

Then:

USART1_TxChar(data_byte);

sends 'A' back.

So Tera Term shows:

A

This is called a UART Echo Program because whatever character you type is immediately transmitted back and displayed.

so first op would be "USART1 Ready\r\n" this one and if give any char there then it would present in data byte and agin this one transmiteed to teram screen right

Exactly right. Let's trace your program execution.

### 1. Program Starts

int main(void)
{
    GPIOA_Init();
    USART1_Init();

    USART1_TxString("USART1 Ready\r\n");

This function sends characters one by one:

'U'
'S'
'A'
'R'
'T'
'1'
' '
'R'
'e'
'a'
'd'
'y'
'\r'
'\n'

Tera Term receives them and displays:

USART1 Ready

### 2. Enter Infinite Loop

while (1)
{
    data_byte = USART1_RxChar();
    USART1_TxChar(data_byte);
}

Now the MCU is waiting here:

data_byte = USART1_RxChar();

until a character arrives on RX (PA10).

### 3. You Type a Character in Tera Term

Suppose you type:

A

Tera Term sends ASCII:

'A' = 0x41

to the STM32 RX pin.

Then:

data_byte = USART1_RxChar();

returns:

data_byte = 'A';

### 4. Character is Echoed Back

Next line:

USART1_TxChar(data_byte);

becomes:

USART1_TxChar('A');

STM32 transmits 'A' back to Tera Term.

Tera Term displays:

A

### Complete Example

Initial display:

USART1 Ready

Now you type:

H

STM32 receives:

data_byte = 'H';

and sends it back.

Screen becomes:

USART1 Ready
H

Then you type:

E

STM32 receives:

data_byte = 'E';

and sends it back.

Screen:

USART1 Ready
HE

Then:

L
L
O

Screen:

USART1 Ready
HELLO

### Important Note

This works as an echo program only if Tera Term is not already locally echoing your keystrokes.

- Local Echo OFF → You see characters because STM32 sends them back.

- Local Echo ON → You see characters immediately from Tera Term itself, even if STM32 doesn't echo them.

So for testing UART echo, keep Local Echo = OFF in Tera Term. Then every character you see proves that:

PC → STM32 RX → data_byte → STM32 TX → PC

worked correctly.

ok fine

Great! 👍

Your program is a classic UART Echo Test:

STM32 boots
    ↓
Displays:
USART1 Ready

You type 'A'
    ↓
USART1_RxChar() receives 'A'
    ↓
data_byte = 'A'
    ↓
USART1_TxChar(data_byte)
    ↓
'A' sent back
    ↓
Tera Term displays 'A'

So the variable:

char data_byte;

acts as a temporary storage location between receiving and transmitting.

### Data Flow

Keyboard
   ↓
Tera Term
   ↓
USB-UART Converter
   ↓
PA10 (USART1_RX)
   ↓
USART1_DR
   ↓
data_byte
   ↓
USART1_DR
   ↓
PA9 (USART1_TX)
   ↓
USB-UART Converter
   ↓
Tera Term Screen

### If you type a whole word

Suppose you type:

HELLO

Then the loop executes 5 times:

| Iteration | Received | data_byte | Transmitted |
| --- | --- | --- | --- |
| 1 | H | H | H |
| 2 | E | E | E |
| 3 | L | L | L |
| 4 | L | L | L |
| 5 | O | O | O |

Result on Tera Term:

USART1 Ready
HELLO

This confirms both:

- ✅ USART RX is working

- ✅ USART TX is working

- ✅ Baud rate is correct

- ✅ PA9/PA10 pin configuration is correct

This echo test is usually the first UART program engineers run on a new board before moving on to interrupts, DMA, or communication with other devices.