2_STM32F401x_Ref_Manual (1).pdf

PDF

CortexM4_STM32_Programming_Manual.pdf

PDF

1_STM32F401x_Datasheet.pdf

PDF

Raayanmini_v4.0_4.1_Schematic.pdf

PDF

based on these explain the chapter Create concise professional notes for: "STM32F401 RCC and Clock Configuration" Cover only: - HSI - HSE - PLL - SYSCLK - AHB/APB clocks - Clock tree basics - Peripheral clock enable - RCC registers - Clock switching - PLL calculation basics Include: - clock tree diagrams - RCC register diagrams - clock configuration examples - bare-metal clock initialization code all hese things

Yes, STM32F401 has an internal 16 MHz clock (HSI).

The 8 MHz I mentioned is not inside the STM32. It is the external crystal oscillator (HSE) mounted on your development board.

Let's separate them clearly:

## Clock Sources Available

### 1. HSI (Internal Clock)

Inside STM32F401:

HSI = 16 MHz

No external components required.

STM32
 ┌──────────┐
 │ HSI      │
 │ 16 MHz   │
 └──────────┘

### 2. HSE (External Clock)

Outside STM32, on the PCB.

From your Raayan Mini schematic:

Y1 = 8 MHz Crystal

connected to:

OSC_IN
OSC_OUT

So:

Crystal = 8 MHz

which becomes:

HSE = 8 MHz

## Why Use 8 MHz if We Already Have 16 MHz?

Because the external crystal is more accurate.

Comparison:

| Clock | Frequency | Accuracy |
| --- | --- | --- |
| HSI | 16 MHz | Medium |
| HSE | 8 MHz Crystal | High |

For applications like:

- UART

- USB

- Precise timers

- Communication protocols

engineers prefer:

HSE + PLL

instead of:

HSI + PLL

## Can PLL Use HSI?

Yes.

Example:

HSI = 16 MHz
        |
       PLL
        |
      84 MHz

No crystal needed.

## Can PLL Use HSE?

Yes.

Example:

HSE = 8 MHz
        |
       PLL
        |
      84 MHz

This is the most common configuration on STM32 boards.

## After Reset What Happens?

When STM32 powers up:

HSI = ON
HSE = OFF
PLL = OFF

Therefore:

CPU = 16 MHz

Initially the MCU runs from HSI.

Only after your code configures RCC:

Enable HSE
Configure PLL
Switch SYSCLK to PLL

does it become:

CPU = 84 MHz

## Real Example

### Case 1: No PLL

Using HSI directly:

HSI = 16 MHz
      |
   SYSCLK

CPU = 16 MHz

### Case 2: Using 8 MHz Crystal + PLL

8 MHz Crystal
       |
      HSE
       |
      PLL
       |
   SYSCLK

CPU = 84 MHz

### Case 3: Using HSI + PLL

HSI 16 MHz
      |
     PLL
      |
   SYSCLK

CPU = 84 MHz

The most important thing to remember is:

16 MHz = Internal clock source (HSI)

8 MHz = External crystal on your board (HSE)

84 MHz = Final CPU clock after PLL

So there are three different frequencies, not one:

HSI      = 16 MHz
HSE      = 8 MHz
SYSCLK   = 84 MHz (after PLL)

The MCU can choose either HSI or HSE as the input to the PLL.

maximum wich it use

If you're asking "At maximum frequency, which clock source does STM32F401 use?"

The answer is:

Maximum CPU Frequency = 84 MHz

To achieve 84 MHz, STM32F401 uses:

HSE (8 MHz Crystal)
        |
       PLL
        |
      84 MHz

or

HSI (16 MHz Internal)
        |
       PLL
        |
      84 MHz

The CPU cannot directly run at 84 MHz from HSI or HSE.

Direct clock frequencies are:

HSI = 16 MHz
HSE = 8 MHz (on your board)

To reach:

84 MHz

you must use PLL.

### In practice

Most STM32 development boards use:

8 MHz Crystal (HSE)
        +
       PLL
        =
      84 MHz

because the crystal is more accurate than the internal RC oscillator.

### Interview answer

Q: What clock source is typically used for maximum STM32F401 speed?

A: External crystal (HSE) fed into PLL, generating 84 MHz SYSCLK for the CPU.

8 MHz HSE
    ↓
   PLL
    ↓
84 MHz SYSCLK

This is the configuration you'll see in most STM32F401 projects.

7\. RCC Registers ================= Most important RCC registers:

here we need to externally means manullay enables the clock haa or automatically taken by the controller

Yes, you must enable it in software if you want to use HSE or PLL.

Let's see what happens after power-up.

### After Reset

STM32 automatically does:

HSI = ON
HSE = OFF
PLL = OFF

So the CPU immediately runs from:

HSI = 16 MHz

No code is needed for this.

### If you want to use the external 8 MHz crystal (HSE)

STM32 does not automatically switch to HSE.

You must enable it yourself:

RCC->CR |= RCC_CR_HSEON;      // Enable HSE

while(!(RCC->CR & RCC_CR_HSERDY));   // Wait until stable

Then:

8 MHz Crystal
     ↓
    HSE

becomes available.

### If you want 84 MHz

You must also enable PLL:

RCC->CR |= RCC_CR_PLLON;

while(!(RCC->CR & RCC_CR_PLLRDY));

Then switch SYSCLK:

RCC->CFGR |= RCC_CFGR_SW_PLL;

Now:

8 MHz Crystal
      ↓
     HSE
      ↓
     PLL
      ↓
   84 MHz

### Why doesn't STM32 automatically use HSE?

Because:

- Some boards don't have a crystal.

- Some boards use 8 MHz crystals.

- Some use 12 MHz.

- Some use external clock generators.

STM32 doesn't know what hardware is connected, so after reset it safely starts with:

HSI = 16 MHz

and waits for your software to configure the clock system.

### Summary

| Clock Source | After Reset |
| --- | --- |
| HSI (16 MHz) | Automatically ON |
| HSE (External Crystal) | OFF |
| PLL | OFF |

So:

HSI → Automatic
HSE → You enable it
PLL → You enable it

This is why one of the first things done in SystemInit() or startup code is:

Enable HSE
Configure PLL
Switch SYSCLK to PLL

to run the STM32F401 at its full 84 MHz speed.

Great question. Once you understand SYSCLK, the entire STM32 clock tree becomes much easier.

### What is SYSCLK?

SYSCLK = System Clock

It is the main clock selected by RCC and supplied to the CPU and bus prescalers.

Think of it as the master clock of the STM32.

Clock Source
     |
     v
   SYSCLK
     |
     +----> AHB (HCLK)
     |
     +----> APB1 (PCLK1)
     |
     +----> APB2 (PCLK2)

### Where does SYSCLK come from?

STM32 can select SYSCLK from:

1. HSI
2. HSE
3. PLL

#### Case 1: HSI selected

HSI = 16 MHz
     |
     v
   SYSCLK

SYSCLK = 16 MHz

#### Case 2: HSE selected

HSE = 8 MHz
     |
     v
   SYSCLK

SYSCLK = 8 MHz

#### Case 3: PLL selected

HSE = 8 MHz
      |
     PLL
      |
      v
   SYSCLK

SYSCLK = 84 MHz

This is the most common configuration.

### Why do we need SYSCLK?

The CPU doesn't directly use HSI, HSE, or PLL.

Instead:

HSI/HSE/PLL
      |
      v
   SYSCLK
      |
      v
     CPU

SYSCLK acts as the selected clock distributed throughout the system.

### Real STM32F401 Example

After reset:

HSI = 16 MHz
SYSCLK = 16 MHz

Later your code does:

Enable HSE
Enable PLL
Select PLL as SYSCLK

Now:

HSE = 8 MHz
     |
    PLL
     |
84 MHz SYSCLK

CPU now runs at:

84 MHz

### Relationship Between SYSCLK, HCLK, PCLK1, PCLK2

Example:

SYSCLK = 84 MHz

AHB Prescaler:

/1

So:

HCLK = 84 MHz

APB1 Prescaler:

/2

So:

PCLK1 = 42 MHz

APB2 Prescaler:

/1

So:

PCLK2 = 84 MHz

Final result:

SYSCLK = 84 MHz
HCLK   = 84 MHz
PCLK1  = 42 MHz
PCLK2  = 84 MHz

### Simple Analogy

Imagine a water system:

HSI/HSE = Water Source

PLL = Pressure Booster

SYSCLK = Main Water Tank

AHB/APB = Distribution Pipes

CPU/Peripherals = Houses

The CPU and peripherals receive clock through SYSCLK and the bus clocks derived from it.

### One-line definition

SYSCLK is the clock selected by RCC (from HSI, HSE, or PLL) and used as the master system clock for the STM32.

how many gpio ports has this

For the STM32F401RBT6 on your Raayan Mini board, the MCU package is LQFP64.

The available GPIO ports are:

| Port | Available Pins |
| --- | --- |
| GPIOA | PA0 – PA15 |
| GPIOB | PB0 – PB15 |
| GPIOC | PC0 – PC15 |
| GPIOD | PD2 only |
| GPIOH | PH0, PH1 (crystal pins) |

So the STM32F401RBT6 contains 5 GPIO ports:

GPIOA
GPIOB
GPIOC
GPIOD
GPIOH

However, not all ports have all 16 pins bonded out in the 64-pin package.

### Pins actually available

GPIOA : 16 pins
GPIOB : 16 pins
GPIOC : 16 pins
GPIOD : 1 pin (PD2)
GPIOH : 2 pins (PH0, PH1)

Total GPIO pins available ≈

16 + 16 + 16 + 1 + 2 = 51 pins

Some of these pins may also be used for:

- SWD Debug (PA13, PA14)

- Crystal Oscillator (PH0, PH1)

- BOOT0

- Reset pin (NRST)

So the number of general-purpose I/O pins available to your application is slightly less than 51.

### GPIO Clock Enable

Before using any GPIO port, you must enable its clock:

RCC->AHB1ENR |= (1<<0);   // GPIOA
RCC->AHB1ENR |= (1<<1);   // GPIOB
RCC->AHB1ENR |= (1<<2);   // GPIOC
RCC->AHB1ENR |= (1<<3);   // GPIOD
RCC->AHB1ENR |= (1<<7);   // GPIOH

Notice that GPIO ports are connected to the AHB1 bus, so their clocks are controlled through RCC_AHB1ENR.

A common interview question is:

Q: How many GPIO ports does STM32F401RBT6 have?

A: Five ports — GPIOA, GPIOB, GPIOC, GPIOD, and GPIOH. However, in the 64-pin package, only PD2 from Port D and PH0/PH1 from Port H are available.

PLLMPLLNPLLPPLLQ Simplified: Input Clock | PLLM | PLLN | PLLP | SYSCLK

what are these

You are asking about:

PLLM
PLLN
PLLP
PLLQ

These are the PLL configuration parameters inside the RCC_PLLCFGR register.

Think of PLL as a machine that first divides, then multiplies, then divides again.

Input Clock
     |
   PLLM
     |
     v
 VCO Input
     |
   PLLN
     |
     v
 VCO Output
     |
  --------
  |      |
 PLLP   PLLQ
  |      |
  v      v
SYSCLK  USB Clock

# 1. PLLM (Input Divider)

First stage of PLL.

Formula:

fVCOIN=fINPLLMfVCOIN​=PLLMfIN​​

Example:

HSE = 8 MHz
PLLM = 8

Then:

VCO Input = 8/8
          = 1 MHz

So PLLM reduces the incoming clock.

# 2. PLLN (Multiplier)

Second stage.

Formula:

fVCOOUT=fVCOIN×PLLNfVCOOUT​=fVCOIN​×PLLN

Example:

VCO Input = 1 MHz
PLLN = 336

Then:

VCO Output = 1 × 336
           = 336 MHz

So PLLN increases the frequency.

# 3. PLLP (SYSCLK Divider)

Third stage.

Used to generate CPU clock (SYSCLK).

Formula:

SYSCLK=fVCOOUTPLLPSYSCLK=PLLPfVCOOUT​​

Example:

VCO Output = 336 MHz
PLLP = 4

Then:

SYSCLK = 336/4
       = 84 MHz

This is why STM32F401 runs at:

84 MHz

# 4. PLLQ (USB/SDIO Divider)

Used for peripherals such as:

USB
SDIO
RNG

Formula:

fUSB=fVCOOUTPLLQfUSB​=PLLQfVCOOUT​​

Example:

VCO Output = 336 MHz
PLLQ = 7

Then:

USB Clock = 336/7
          = 48 MHz

USB requires exactly:

48 MHz

which is why PLLQ exists.

# Complete Example (Most Common)

Suppose your board has:

HSE = 8 MHz

Configuration:

PLLM = 8
PLLN = 336
PLLP = 4
PLLQ = 7

Calculation:

8 MHz / 8 = 1 MHz

1 MHz × 336 = 336 MHz

336 MHz / 4 = 84 MHz SYSCLK

336 MHz / 7 = 48 MHz USB Clock

Final:

SYSCLK = 84 MHz
USB    = 48 MHz

# Easy Memory Trick

PLLM = M = Make Smaller
        (divide input)

PLLN = N = Multiply
        (increase frequency)

PLLP = P = Processor Clock
        (SYSCLK)

PLLQ = Q = USB Clock

So whenever you see:

RCC->PLLCFGR

remember:

PLLM → divide
PLLN → multiply
PLLP → CPU clock
PLLQ → USB clock

That's the core idea behind STM32F401 PLL configuration.

Top of Form

Bottom of Form